## LEGAL DESCRIPTION

A PARCEL OF LAND BEING A PORTION OF THE SOUTHWEST QUARTER (SW 1/4) AND A PORTION OF THE SOUTHWEST QUARTER OF THE NORTHWEST QUARTER (SW1/4 NW1/4) OF SECTION 33, TOWNSHIP 14 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, AND A PORTION OF LOT 1, BLOCK 1 OF HASSLER & BATES SUBDIVISION NO. 3 RECORDED JANUARY 10, 1977 UNDER RECEPTION NO. 291096 IN PLAT BOOK D-3 AT PAGE 26 OF THE OFFICIAL RECORDS SAID EL PASO COUNTY, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE EAST-WEST CENTERLINE OF SECTION 33, TOWNSHIP 14 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN BEING MONUMENTED AT THE EAST QUARTER CORNER BY A 2-1/2" ALUMINUM SURVEYORS CAP ESTABLISHED BY PLS 30118 AND AT THE WEST QUARTER CORNER BY A 3-1/4" ALUMINUM SURVEYORS CAP ESTABLISHED BY LS 22573, IS ASSUMED TO BEAR SOUTH 89°12'26"WEST, A DISTANCE OF 5247.11 FEET;

COMMENCING AT SAID EAST QUARTER CORNER OF SAID SECTION 33;

THENCE SOUTH 89°12'26" WEST ON THE EAST-WEST CENTERLINE OF SAID SECTION 33, A DISTANCE OF 2279.88 FEET TO A POINT ON THE NORTHERLY BOUNDARY OF LOT 1, BLOCK 1 AS PLATTED IN SAUD HASSLER & BATES SUBDIVISION NO. 3, SAID POINT ALSO BEING ON THE SOUTHERLY BOUNDARY LINE OF COLORADO SPRINGS YOUTH SPORTS COMPLEX AS RECORDED APRIL 2, 1999 UNDER RECEPTION NO. 99050902 OF SAID RECORDS, SAID POINT ALSO BEING THE POINT OF BEGINNING;

THENCE SOUTH 11°59'09" WEST, A DISTANCE OF 522.12 FEET;

THENCE SOUTH 00°47'59" EAST A DISTANCE OF 672.85 FEET;

THENCE SOUTH 89°12'26" WEST A DISTANCE OF 400.00 FEET;

THENCE NORTH 00°47'34" WEST A DISTANCE OF 252.43 FEET;

THENCE SOUTH 89°12'26" WEST A DISTANCE OF 201.81 FEET TO A POINT ON A CURVE, WHOSE CENTER BEARS SOUTH 47°08'42" WEST;

THENCE ON A CURVE TO THE RIGHT THROUGH A CENTRAL ANGLE OF 159°08'08", HAVING A RADIUS OF 65.00 FEET FOR AN ARC DISTANCE OF 180.53 FEET TO A POINT OF REVERSE CURVE;

THENCE ON A CURVE TO THE LEFT THROUGH A CENTRAL ANGLE OF 73°52'21", HAVING A RADIUS OF 25.00 FEET FOR AN ARC DISTANCE OF 32.23 FEET TO A POINT OF REVERSE CURVE;

THENCE ON A CURVE TO THE RIGHT THROUGH A CENTRAL ANGLE OF 13°43'58", HAVING A RADIUS OF 682.50 FEET FOR AN ARC DISTANCE OF 163.58 FEET TO THE SOUTHEAST CORNER OF VINEYARD PARK DRIVE, AS DEDICATED TO THE PUBLIC IN T5 @ COLORADO SPRINGS FILING NO. 2 RECORDED JUNE 8, 2026 UNDER RECEPTION NO. 216713783 OF SAID RECORDS;

THENCE ON THE NORTHEASTERLY AND NORTHERLY BOUNDARY LINES OF SAID T5 @ COLORADO SPRINGS FILING NO. 2 THE FOLLOWING TWO COURSES:

1. 	THENCE NORTH 33°51’13” WEST A DISTANCE OF 670.65 FEET;

2. 	THENCE SOUTH 89°33’09” WEST A DISTANCE OF 370.59 FEET TO A POINT ON THE EASTERLY BOUNDARY LINE OF SIPILOVIC SUBDIVISION RECORDED JULY 15, 1998 UNDER RECEPTION NO. 98098685 OF SAID RECORDS;

THENCE NORTH 00°16’22” WEST, ON SAID EASTERLY BOUNDARY LINE OF LOT 2, A DISTANCE OF 191.65 FEET TO A POINT ON THE SOUTHWESTERLY BOUNDARY LINE OF LOT 1 AS PLATTED IN HARRISON PARK FILING NO. 17 RECORDED JULY 7, 1999 UNDER RECEPTION NO. 99109078 OF SAID RECORDS;

THENCE ON THE SOUTHWESTERLY AND EASTERLY BOUNDARY LINESOF SAID LOT 1THE FOLLOWING TWO (2) COURSES:

1. 	THENCE SOUTH 48°54’33” EAST A DISTANCE OF 53.30 FEET;

2. 	THENCE NORTH 00°16’22” WEST A DISTANCE OF 420.59 FEET TO A POINT ON SAID EAST-WEST CENTERLINE OF SECTION 33, SAID POINT ALSO BEING ON THE SOUTHERLY BOUNDARY LINE OF STOOKS SUBDIVISION RECORDED APRIL 20, 2006 UNDER RECEPTION NO. 206712297 OF SAID RECORDS;

THENCE NORTH 89°12'26" EAST, ON SAID EAST-WEST CENTERLINE, AND SAID SOUTHERLY BOUNDARY LINE OF SAID STOOKS SUBDIVISION, OF TRANEX SUBDIVISION FILING NO. 1 AS RECORDED JUNE 11, 2002 UNDER RECEPTION NO. 202094392 OF SAID RECORDS AND OF HARRISON PARK FILING NO. 11, AS RECORDED SEPTEMBER 6, 1984 UNDER RECEPTION NO. 1158884 IN PLAT BOOK W-3 AT PAGE 128 OF SAID RECORDS, A DISTANCE OF 939.50 FEET TO THE SOUTHEAST CORNER OF SAID HARRISON PARK FILING NO. 11;

THENCE NORTH 16°52'09” WEST ON THE EASTERLY BOUNDARY LINE OF SAID HARRISON PARK FILING NO. 11, A DISTANCE OF 364.57 FEET TO A POINT ON THE SOUTHERLY RIGHT-OF-WAY LINE OF EXECUTIVE CIRCLE DEDICATED TO THE PUBLIC IN HARRISON PARK FILING NO. 8, RECORDED JANUARY 13, 1981 UNDER RECEPTION NO. 737017 IN PLAT BOOK M-3 AT PAGE 100 OF SAID RECORDS;

THENCE NORTH 73°07'51” EAST ON SAID SOUTHERLY RIGHT-OF-WAY LINE, A DISTANCE OF 60.00 FEET TO A POINT ON THE WESTERLY BOUNDARY LINE OF SAID COLORADO SPRINGS YOUTH SPORTS COMPLEX;

THENCE ON THE WESTERLY AND SOUTHERLY BOUNDARY OF SAID COLORADO SPRINGS YOUTH SPORTS COMPLEX, THE FOLLOWING TWO COURSES:

1.	THENCE SOUTH 16°52'09” EAST A DISTANCE OF 381.86 FEET TO A POINT ON SAID EAST-WEST CENTERLINE OF SECTION 33;

2. 	NORTH 89°12'26” EAST  ON SAID EAST-WEST CENTERLINE OF SECTION 33, A DISTANCE OF 639.05 FEET TO THE POINT OF BEGINNING.

THE ABOVE DESCRIPTION PRODUCES AN AREA OF 33.790 ACRES OF LAND, MORE OR LESS.

JOHN L. BAILEY, P.L.S. NO. 19586

PREPARED FOR AND ON BEHALF OF

CLASSIC CONSULTING ENGINEERS AND SURVEYORS