SITE INFORMATION

PARCEL ID: 6232101002

PLAT NO: R10026

PARCEL ADDRESS: 1625 SPRINGCREST RD

ZONING: BP/CR HR

LEGAL DESCRIPTION:

LOT 1 BRIARGATE BUSINESS CAMPUS FIL NO 20

AREA: 7.93 ACRES

A TRACT OF LAND BEING A PORTION OF THE NORTHEAST QUARTER OF SECTION 32, TOWNSHIP 12 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, CITY OF COLORADO SPRINGS, EL PASO COUNTY, COLORADO, BEING DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE EAST LINE OF THE NORTHEAST QUARTER OF SECTION 32, TOWNSHIP 12 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, THE EAST QUARTER CORNER OF SAID SECTION 32 BEING MONUMENTED WITH A 3-1/4" BLM BRASS CAP AND THE NORTHEAST CORNER OF SAID SECTION 32 BEING MONUMENTED WITH A 3-1/4" BLM BRASS CAP, IS ASSUMED TO BEAR N00°21'28"W, A DISTANCE OF 2649.65 FEET.

COMMENCING AT THE NORTHWESTERLY CORNER OF BRIARGATE BUSINESS CAMPUS FILING NO. 10, AS RECORDED IN PLAT BOOK E-5 AT PAGE 55, EL PASO COUNTY, COLORADO, SAID POINT BEING A POINT ON THE EASTERLY RIGHT-OF-WAY LINE OF COLORADO STATE HIGHWAY 83, AS RECORDED IN BOOK 5220 AT PAGE 1195, SAID POINT BEING THE POINT OF BEGINNING; THENCE NORTHERLY, ON SAID EASTERLY RIGHT-OF-WAY LINE, THE FOLLOWING NINE (9) COURSES:

- N61°52'51"W, A DISTANCE OF 30.00 FEET TO A POINT OF CURVE;

- ON THE ARC OF A CURVE TO THE RIGHT, HAVING A DELTA OF 69°45'35", A RADIUS OF 170.00 FEET, A DISTANCE OF 206.98 FEET TO A POINT OF TANGENT;

- N07°52'44"E, A DISTANCE OF 108.56 FEET TO A POINT OF CURVE;

- ON THE ARC OF A CURVE TO THE LEFT, HAVING A DELTA OF 20°10'32", A RADIUS OF 1501.89 FEET, A DISTANCE OF 528.86 FEET TO A POINT OF TANGENT;

- N12°17'48"W, A DISTANCE OF 33.19 FEET;

- N14°35'48"W, A DISTANCE OF 210.38 FEET;

- N11°43'30"W, A DISTANCE OF 87.58 FEET;

- N34°32'02"E, A DISTANCE OF 30.41 FEET;

- N00°22'17"W, A DISTANCE OF 12.00 FEET TO A POINT BEING 30.00 FEET SOUTH OF THE NORTH LINE OF THE NORTHEAST QUARTER OF SAID SECTION 32;

THENCE N89°37'08"E, ALONG A LINE 30.00 FEET SOUTH OF AND PARALLEL WITH THE NORTH LINE OF SAID NORTHEAST QUARTER, A DISTANCE OF 270.80 FEET TO THE NORTHERLY CORNER OF A TRACT OF LAND DESCRIBED IN BOOK 5442 AT PAGE 1471; THENCE S15°08'08"E, ON THE EASTERLY LINE OF SAID TRACT, A DISTANCE OF 773.76 FEET; THENCE S12°35'32"W, A DISTANCE OF 136.04 FEET; THENCE S44°42'27"W, A DISTANCE OF 329.40 FEET; THENCE S25°36'41"W, A DISTANCE OF 55.00 FEET TO A POINT ON THE NORTHERLY RIGHT-OF-WAY LINE OF BRIARGATE PARKWAY, AS PLATTED IN SAID BRIARGATE BUSINESS CAMPUS FILING NO. 10; THENCE N61°55'24"W, ON SAID NORTHERLY RIGHT-OF-WAY LINE, A DISTANCE OF 5.77 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 7.9331 ACRES.