LEGAL DESCRIPTION:

A TRACT OF LAND BEING A PORTION OF THE NORTHWEST QUARTER OF SECTION 18, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6TH P.M., COUNTY OF EL PASO, STATE OF COLORADO, MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE COURSE ON THE NORTHERLY BOUNDARY LINE OF THE WESTERLY PORTION OF THE TRACT OF LAND DESCRIBED IN WARRANTY DEED RECORDED UNDER RECEPTION NO. 206150812 OF THE RECORDS OF THE EL PASO COUNTY, COLORADO, BEING MONUMENTED AT THE WEST END BY A REBAR AND YELLOW PLASTIC CAP STAMPED “WKC&ASSOC PLS4842” 0.1’ BELOW GRADE, AND AT THE EAST END BY A REBAR AND YELLOW PLASTIC CAP STAMPED “WKC&ASSOC PLS4842” 0.1’ BELOW GRADE. ASSUMED TO BEAR S89°44'22"E FEET (DEED BEARING S89°18’55”E) A DISTANCE OF 1,424.82 FEET.

COMMENCING AT THE WESTERLY END OF THE COURSE ON THE NORTHERLY BOUNDARY LINE OF THE WESTERLY PORTION OF THE TRACT OF LAND DESCRIBED IN THE WARRANTY DEED RECORDED UNDER RECEPTION NO. 206150812 IN, WHICH BEARS S89°44’22”E (DEED BEARING S89°18’55”E) A DISTANCE OF 1,424 FEET, SAID POINT ALSO BEING THE POINT OF BEGINNING, THENCE S89°44'22"E ON SAID BOUNDARY LINE A DISTANCE OF 1,424.82 FEET; THENCE S21°28'11"E A DISTANCE OF 450.34 FEET; THENCE S04°06'20"E A DISTANCE OF 115.05 FEET: THENCE S72°42'15"E A DISTANCE OF 27.29 FEET TO A POINT ON CURVE, THENCE ON AN ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS S72°42’15”E, HAVING A DELTA OF 17°06’50”, A RADIUS OF 330.00 FEET TO A POINT OF TANGENT: THENCE S00°10’55”W A DISTANCE OF 55.44 FEET, THENCE S00°15'59"W A DISTANCE OF 40.00 FEET TO A POINT ON THE SOUTH LINE OF THE NORTH HALF OF SAID SECTION 18 AND THE SOUTHERLY LINE OF SAID TRACT OF LAND RECORDED UNDER RECEPTION NO. 206150812; THENCE CONTINUING ON THE BOUNDARY OF SAID TRACT OF LAND, THE FOLLOWING TWO (2) COURSES;

- N89°44'01"W A DISTANCE OF 1,603.61 FEET TO THE WEST QUARTER CORNER OF SAID SECTION 18;

- N00°23'41"W ON WEST LINE OF THE NORTHWEST QUARTER OF SAID SECTION 18, A DISTANCE 733.50 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 1,134,576 SQUARE FEET OR 26.046 ACRES.