| Department of Public Works |  |  |
| --- | --- | --- |
| Kevin Mastin, Executive Director 719-520-6460 Public Works 3275 Akers Dr. Colorado Springs, CO 80922 www.ElPasoCo.com |  | Board of County Commissioners Holly Williams, District 1 Carrie Geitner, District 2 Stan VanderWerf, District 3 Longinos Gonzalez, Jr., District 4 Cami Bremer, District 5 |
| PRE-CONSTRUCTION MEETING FILE#SF2010 DATE: 2/21/2023 Development: Prairie Ridge Developer: Clovis Point CO, LLC Engineer/Firm: TBD Developer Contact: Alexander Kuhnke Inspector Contact: Spencer Pirzadeh (719) 425-7088 SpencerPirzadeh2@elpasoco.com Stormwater Contact: TBD APPROVAL CHECKLIST: REQUIREMENT APPROVED FORTHCOMING / NOTE N/A Early Grading Construction Plans; 8/2/2022. Preliminary Final Drainage Report; 8/2/2022. Financial Assurance Estimate; 8/2/2022. Surety Posted; TBD; Collect At Pre-Con. CDPHE Construction Permit (APEN); date.. EPC Health Dept. Air Quality Permit; TBD. SWMP; TBD. Colorado Discharge Permit; date.; Collect Before NTP. Initial BMP Installation / Inspection SW; date.; Must be completed first.. EPC DPW Right-of-Way Permit; date.. ESQCP; 6/18/2022. Construction Schedule; date.. Construction Permit Fee*. ALL APPLICABLE FEDERAL, STATE AND LOCAL PERMITS, INSURANCE AND BONDING ARE TO BE OBTAINED PRIOR TO START OF CONSTRUCTION. WHEN ALL ITEMS (Subdivision Improvements) HAVE BEEN COMPLETED: - Submit written request for inspection punch list from EPC PCD. - Provide letters of acceptance for perpetual maintenance from all utility’s providers. - Provide design engineer’s signed and stamped certification letter of substantial compliance. - Upon satisfactory completion of punch list items and posting of applicable Defect Warranty surety a request for EPC DPW maintenance acceptance may be submitted to EPC. - Warranty period commences on date of preliminary acceptance by EPC BoCC. NOTICE TO PROCEED: Click a date. SIGNED BY:________________________________ El Paso County PCD Representative *Construction permit fees cover all inspections and fees through final acceptance.. PRE-CONSTRUCTION MEETING FILE#SF2010 DATE: 2/21/2023 Pre-construction Questionnaire: - Who will be your main GC and contact? - Estimated start time and completion time? - Have you submitted street lighting plans? - Testing company? - Are initial BMPs installed, when will they be installed? - Discuss conditions or approvals that didn’t make it into the SIA? - Approved set of plans being used. Any changes require PCD approval prior to construction. - Fees are to be paid during the pre-con. Fees included are for the construction permit, preliminary, and final acceptance. Additional fees may be applied. - Concrete must be 4500 PSI. PRE-CONSTRUCTION MEETING FILE# SF2010 DATE: 2/21/2023 Testing Requirements: Requirements: - Add EPC PCD to the distribution list for all testing reports, in a timely fashion, necessary to comply with the ECM Standards. A summary sheet of typical requirements is included with this document. - Minimum testing intervals of 250 LF applicable to trenches, 250 Lane Ft. subgrade. - Testing of trenches / embankments not to exceed 2 foot depth intervals. - Provide pavement design along with HBP mix design to EPC PCD for review / approval at least 10 working days prior to anticipated date of placement. - Contact EPC PCD for subgrade inspection / proof roll prior to placement of gravel or pavement. Signing and Striping Notes: - All signs and pavement markings shall be in compliance with the current Manual on Uniform Traffic Control Devices (MUTCD). - Removal of existing pavement markings shall be accomplished by a method that does not materially damage the pavement. The pavement markings shall be removed to the extent that they will not be visible under day or night conditions. At no time will it be acceptable to paint over existing pavement markings. - Any deviation from the striping and signing plan shall be approved by El Paso County Planning and Community Development. - All signs shown on the signing and striping plan shall be new signs. Existing signs may remain or be reused if they meet current El Paso County and MUTCD standards. - Street name and regulatory stop signs shall be on the same post at intersections. - All removed signs shall be disposed of in a proper manner by the contractor. - All street name signs shall have “D” series letters, with local roadway signs being 4” upper-lower case lettering on 8” blank and non-local roadway signs being 6” lettering, upper-lower case on 12” blank, with a white border that is not recessed. Multi-lane roadways with speed limits of 40 mph or higher shall have 8” upper-lower case lettering on 18” blank with a white border that is not recessed. The width of the non-recessed white borders shall match page 255 of the 2012 MUTCD "Standard Highway Signs" - All traffic signs shall have a minimum High Intensity Prismatic grade sheeting. - All local residential street signs shall be mounted on a 1.75” x 1.75” square tube sign post and stub post base. For other applications, refer to the CDOT Standard S-614-8 regarding use of the P2 tubular steel post slipbase design. - All signs shall be single sheet aluminum with 0.100” minimum thickness. - All limit lines/stop lines, crosswalk lines, pavement legends, and arrows shall be a minimum 125 mil thickness preformed thermoplastic pavement markings with tapered leading edges per CDOT Standard S-627-1. Word and symbol markings shall be the narrow type. Stop bars shall be 24” in width. Crosswalks lines shall be 12” wide and 8’ long per CDOT S-627-1. - All longitudinal lines shall be a minimum 15 mil thickness epoxy paint. All non-local residential roadways shall include both right and left edge line striping and any additional striping as required by CDOT S-627-1. - The contractor shall notify El Paso County PCD (719) 520-6819 prior to and upon completion of signing and striping. - The contractor shall obtain a work in the right of way permit from the El Paso County Department of Works (DPW) prior to any signage or striping work within an existing El Paso County roadway. J.5. - TESTING SUMMARY Table J-3 summarize the testing requirements outlined in Appendix J. Table J-3. Summary Minimum Testing Requirements No.; Item; Type Of Test; Minimum Frequency; Min. #; Act. #. 1; Utilities: Water, Sewer, Gas, Electric, Phone and Cable Trenches; Moisture/Density; 1 per every 250 L.F. or fraction thereof, every 2' elevation. Each structure (manhole, valve, etc.) every 2' elevations. This requirement also applies to every service.. 2; Storm Sewer Trench; Moisture/Density; Same as above. Each structure (manhole, inlet, etc.) every 2' elevations.. 3; Inlets Concrete Testing; Air Slump; First load or until compliance. Minimum 1 per day.. Cylinders; 1 set (4) per project for every 100 C.Y. or fraction thereof.. Steel; Visual and Documentation. 4; Curb and Gutter; Moisture/Density; 1 every 250 feet or a minimum of 3 tests, whichever is greater.. Proof-Roll; All final subgrade.. Air/Slump; First load or until compliance. Minimum 1 per every 2,000 L.F. or fraction thereof.. Cylinders; 1 set (4) per project every 2,000 L.F. or fraction thereof.. 5; Sidewalk; Moisture/Density; 1 per every 250 L.F. or fraction thereof.. Air/Slump; First load or until compliance. Minimum 1 per every 1,000 S.Y. or fraction thereof.. Cylinders; 1 set (4) per project 1,000 S.Y. or fraction thereof.. 6; Roadway Subgrade: Fills and Cuts; Moisture/Density; 1 per every 500 Lane Ft. or fraction thereof, every 2' elevation. Cuts to be scarified to a depth of 1', recondition as needed and recompact.. All final subgrade.. 7; Roadway (Concrete); Air/Slump; First 3 loads or until compliance, then every 1,250 C.Y. or fraction thereof.. Cylinders; 1 set (4) per project for every 1,250 C.Y. or fraction thereof.. 8; Roadways (Base Coarse); Moisture/Density; 1 per 500 Lane Ft. or fraction thereof.. Gradation & Atterberg Limits; 1 per 2,000 Tons or fraction thereof.. Proof-Roll; Subgrade prior to Base Course placement. Base Course prior to placement of asphalt.. 9; Roadways (Asphalt); Density; 1 per 500 Lane Ft. or fraction thereof, per lift.. Extraction/Gradation; 1 per 1,000 Tons or fraction thereof.. 10; Roadways (Asphalt & Concrete); Cores; 1 per 1,000 Lane Ft. or fraction thereof.. Profil-O-Graph; Minor Arterial classification and above.. |