LEGAL DESCRIPTION: FLYING HORSE NORTH FILING NO. 8

A TRACT OF LAND BEING A PORTION OF WEST HALF AND A PORTION OF THE SOUTHEAST QUARTER OF SECTION 35, TOWNSHIP 11 SOUTH, RANGE 65 WEST, AND A PORTION OF EAST HALF AND A PORTION OF THE SOUTHWEST QUARTER OF SECTION 36, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS:

THE SOUTH LINE OF THE SOUTHEAST QUARTER OF SECTION 36, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, BEING MONUMENTED AT THE EASTERLY END BY A 3-1/4” ALUMINUM CAP STAMPED "RICHARD M PEARSON  R66W R65W S36 S31 T11S S1 S6 T12S PLS 15686” 0.5’ BELOW GRADE, AND AT THE WESTERLY  END BY A 3-1/4” ALUMINUM CAP STAMPED "LADD ENGINEERING T11S R66W ¼ S 36 S1 T12S 2000 PLS 9132" 0.3’ ABOVE GRADE. ASSUMED TO BEAR S89°20'59"W A DISTANCE OF 2,674.51 FEET.

COMMENCING AT THE SOUTHEAST CORNER OF LOT 12, FLYING HORSE NORTH FILING NO. 3, AS RECORDED UNDER RECEPTION NUMBER 224715365, SAID POINT BEING THE POINT OF BEGINNING; THENCE ON SAID BOUNDARY OF SAID FLYING HORSE FILING NO. 3, THE FOLLOWING FIFTEEN (15) COURSES:

- N02°21’44”W A DISTANCE OF 120.00 FEET;

- S88°18’15”E A DISTANCE OF 1705.84 FEET;

- N19°16’02”E A DISTANCE OF 386.88 FEET;

- N43°30’36”E A DISTANCE OF 161.72 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS N17°38’34”E, HAVING A DELTA OF 105°57’32” A RADIUS OF 183.50 FEET A DISTANCE OF 339.35 FEET TO A POINT OF TANGENT;

- N01°41’01”E A DISTANCE OF 409.04 FEET;

- N10°53’40”E A DISTANCE OF 511.85 FEET;

- N11°16’18”E A DISTANCE OF 794.70 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS N69°45’19”W, HAVING A DELTA OF 113°41’16” A RADIUS OF 80.00 FEET, A DISTANCE OF 158.74 FEET TO A POINT ON CURVE;

- N11°15’44”E A DISTANCE OF 449.78 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 19°27’35", A RADIUS OF 180.00 FEET, A DISTANCE OF 61.13 FEET TO A POINT OF TANGENT;

- N30°43’19”E A DISTANCE OF 748.70 FEET;

- N83°30’56”E A DISTANCE OF 43.73 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE RIGHT WHOSE CENTER BEARS S78°35’26”E, HAVING A DELTA OF 54°21’11", A RADIUS OF 330.00 FEET, A DISTANCE OF 313.05 FEET TO A POINT OF TANGENT;

- N65°45’45”E A DISTANCE OF 64.75 FEET TO A POINT ON THE SOUTHERLY BOUNDARY LINE OF FLYING HORSE NORTH FILING NO. 5, AS RECORDED UNDER RECEPTION NUMBER 225715504;

THENCE S77°19’50”E ON SAID SOUTHERLY BOUNDARY LINE A DISTANCE OF 99.91 FEET TO A POINT ON THE BOUNDARY LINE OF TRACT M, FLYING HORSE NORTH FILING NO. 1, AS RECORDED UNDER RECEPTION NUMBER 218714238; THENCE ON THE BOUNDARY LINE OF SAID TRACT M, THE FOLLOWING NINE (9) COURSES:

- S65°45’45”W A DISTANCE OF 144.64 FEET TO A POINT OF CURVE;

- ON THE ARC OF A CURVE TO THE LEFT HAVING A DELTA OF 54°25’41", A RADIUS OF 270.00 FEET A DISTANCE OF 256.49 FEET TO A POINT ON CURVE;

- N89°19’51”E A DISTANCE OF 44.51 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS N78°15’26”E, HAVING A DELTA OF 35°23’13", A RADIUS OF 222.71 FEET, A DISTANCE OF 137.55 FEET TO A POINT OF TANGENT;

- S47°07’47”E A DISTANCE OF 236.98 FEET;

- S52°20’15”E A DISTANCE OF 614.62 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS N37°35’23”E, HAVING A DELTA 32°49’43”, A RADIUS OF 180.00 FEET, A DISTANCE OF 103.13 FEET TO A POINT OF TANGENT;

- S85°14’20”E A DISTANCE OF 773.82 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS N03°54’09”E, HAVING A DELTA OF 70°52’24", A RADIUS OF 74.72 FEET DISTANCE OF 92.42

FEET TO A POINT ON CURVE, SAID POINT BEING ON THE BOUNDARY LINE OF SAID FLYING HORSE NORTH FILING NO. 5;

THENCE ON THE BOUNDARY LINE OF SAID FLYING HORSE NORTH FILING NO. 5, THE FOLLOWING TWO (2) COURSES:

- S88°25’47”E A DISTANCE OF 316.03 FEET;

- S01°31’19”W A DISTANCE OF 104.22 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 89°19’06”, A RADIUS OF 270.00 FEET A DISTANCE OF 420.90 FEET TO A POINT OF TANGENT; THENCE N89°09’34”W A DISTANCE OF 310.47 FEET; THENCE S00°50’26”W A DISTANCE OF 60.00 FEET; THENCE S89°09’34”E A DISTANCE OF 120.00 FEET; THES S00°50’26”W A DISTANCE OF 452.47 FEET TO A POINT OF CURVE; THENCE ON THE ARC OF CURVE TO THE RIGHT HAVING A DELTA OF 30°26’22”, A RADIUS OF 270.00 FEET A DISTANCE OF 143.44 FEET TO A POINT ON CURVE; THENCE S58°43’12”E A DISTANCE OF 60.00 FEET; THENCE S41°03’30”E A DISTANCE OF 193.08 FEET; THENCE S00°36’03”E A DISTANCE OF 371.50 FEET TO A POINT ON THE NORTHERLY LINE OF PALMER DIVIDE AS RECORDED UNDER RECEPTION NUMBER 205084216; THENCE S89°24’17”W ON SAID NORTHERLY LINE A DISTANCE OF 585.65 FEET TO THE NORTHWEST CORNER OF SAID PALMER DIVIDE; THE NORTHERLY END OF BOUNDARY LINE AGREEMENT RECORDED UNDER RECEPTION NUMBER 204188565; THENCE S02°42'03"W, ON SAID LINE DESCRIBED IN A BOUNDARY LINE AGREEMENT RECORDED UNDER RECEPTION NO. 204188565 A DISTANCE OF 1,330.04 FEET TO A POINT ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 31; THENCE S89°35'20"W, ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 31, A DISTANCE OF 1329.35 FEET TO THE SOUTHWEST CORNER OF SAID SECTION 31 SAID POINT ALSO BEING THE SOUTHEAST CORNER OF SAID SECTION 36; THENCE S89°20’59”W ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 36, A DISTANCE OF 2,674.51 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 6,944,974 SQUARE FEET OR 159.435 ACRES.