LEGAL DESCRIPTION:   WATERBURY LIFT STATION BOUNDARY

A TRACT OF LAND BEING A PORTION OF THE SOUTHWEST QUARTER OF SECTION 28, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6TH P.M., EL PASO COUNTY, STATE OF COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: 	THE NORTH LINE OF SECTION 28, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, BEING MONUMENTED AT EACH END BY A 3-1/4” ALUMINUM SURVEYOR'S CAP STAMPED “PSINC LS 30087” AND ASSUMED TO BEAR S89°47'04”E A DISTANCE OF 5,285.07 FEET.

COMMENCING AT THE NORTHWEST CORNER OF SECTION 28, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6 TH P.M., THENCE S28°29’20”E A DISTANCE OF 5,076.22 FEET TO THE POINT OF BEGINNING; THENCE S80°04’43”E A DISTANCE OF 118.34 FEET; THENCE N77°29’03”E A DISTANCE OF 142.11 FEET TO A POINT IN THE EAST LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 28; THENCE S00°26’21”E ON SAID EAST LINE A DISTANCE OF 150.18 FEET TO A POINT  ON CURVE, SAID POINT BEING ON THE BEING ON THE NORTHERLY LINE OF A SPECIAL WARRANTY DEED RECORDED AT RECEPTION NUMBER 208025323 OF THE RECORDS OF EL PASO COUNTY; THENCE ON SAID NORTHERLY BOUNDARY LINE, ON THE ARC OF A CURVE TO THE LEFT, WHOSE CENTER BEARS S45°40’03”E, HAVING A DELTA OF 05°50’23”, A RADIUS OF 640.00 FEET A DISTANCE OF  65.23 FEET TO A POINT OF ON CURVE; THENCE N51°31’43”W A DISTANCE OF 284.40 FEET; THENCE N38°29’42”E A DISTANCE OF 15.00 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 27,349 SQUARE FEET OR 0.628 ACRES