EA File No. EA 2599 Pallas and David’s Project

Engineering Meeting Notes

| EA Meeting Date/Time: | Wednesday, 9/10/2025 11:00 |
| --- | --- |
| Parcel Number: | 300000005 |
| Address./Plat No./Acre: | 38435 JUDGE ORR RD // not platted // 326.39 |
| Project Manager/Phone: | Kari Parsons (719) 520-6306 |
| Development Services Engineer/Phone: | Ed Schoenheit (719) 520-6813 |
| Stormwater Engineer/Phone | Zachary Jannusch (719) 500-3170 |
| Application/Land Use Type: | Rezone / Site Development Pan |

## IMPORTANT NOTE:

These Engineering meeting notes are based on the information provided by the Applicant and reasonable preliminary research.  The County requirements and policies in effect at the time of the meeting may change prior to the project submittal date, and the requirements and policies in effect at the date of submittal shall apply.  Based on the applicant-provided information and preliminary research, these notes are the best estimate of the requirements expected to be met by the Applicant.  The actual requirements may change based on project revisions, new information, or constraints that were unavailable or overlooked at the time of the Early Assistance meeting.  It is solely the Applicant’s responsibility to research and be familiar with state and federal laws and permitting requirements; the County’s regulations, codes and criteria; the requirements of other applicable local, state or federal agencies; and any governing documents that apply to the project, including previous drainage and transportation studies and previous land use approvals and conditions. Reference links are provided at the end of this document.

REQUEST

Legal Description for the parcel number is: N2 EXRD, W/MR SEC 4-13-60 326 acres of undeveloped agricultural land Will it need to be rezoned to residential, or part of the acreage, as we are building our house on this property? We already have our builder and the modular house. He will be pulling all the permits for the build. Building an animal rescue Re-zoning for RV's

Drainage Impacts

Requirements

| Documents |  | Design Elements |  |
| --- | --- | --- | --- |
| ☐ | Drainage Basin Planning Study (DBPS) | ☒ | Full Spectrum Detention |
| ☐ | Master Development Drainage Plan (MDDP) | ☒ | Permanent Water Quality |
| ☒ | Preliminary/Final Drainage Report/Letter | ☐ | Downstream Conveyance Analysis (to include possible offsite improvements) |
| ☒ | Financial Assurance Estimate (FAE) Form | ☐ | Floodplain Impacts |
| ☒ | Builder’s Erosion and Stormwater Quality Control Permit (BESQCP) | ☐ | Public Improvements |
| ☒ | Erosion and Stormwater Quality Control Permit (ESQCP) |  |  |
| ☒ | Grading & Erosion Control (GEC) Plan & GEC Checklist | Miscellaneous |  |
| ☒ | Stormwater Management Plan (SWMP) & SWMP Checklist | ☐ | Construction Activity Permit/ CDPHE Discharge Permit/CDPHE APEN Fee |
| ☒ | PCM Applicability Form | ☐ | Special Districts/Water Quality Authority Areas |
| ☒ | PCM Maintenance Agreement and O&M Manual |  |  |
| ☒ | MHFD-Detention Basin Design Workbook or SDI Sheet |  |  |
| ☐ | Pre-Subdivision Site Grading Form |  |  |
| ☐ | Deviation Request |  |  |

| Basin Name/Studied | Basin Fee* | Bridge Fee* |
| --- | --- | --- |
| LITTLE HORSE CREEK/ | 0.0 | 0.0 |

* Per impervious acre, collected prior to recordation of Final Plat.

- A 25% fee reduction applies for Low Density Lots (2.5 acres and greater lot sizes).

Drainage Notes:

- The flood map for the selected area is number08041C0675G, effective on12/7/2018

- No drainage documents required for rezone.

- Drainage documents will required with the site development plan with a disturbance greater than acre.

Land disturbance of 1 acre or greater or 500 cy triggers:

- ESQCP

- GEC plans, GEC checklist, SWMP, BMP agreement, FAE, PBMP app, and SWMP checklist

- Full spectrum detention and water quality, SDI sheet

  - Exclusions can be found in our PCM Applicability Form and ECM Appendix I.7

    - Note that the ECM Appendix I.7.1.B.5 Large Lot Single Family Sites exclusion can apply to Agriculturally zoned lands, but the lot utilizing the exclusion cannot exceed 10% imperviousness. For sites with 10-20% imperviousness this exclusion can apply when a study specific to the watershed and/or MS4 shows that expected soil and vegetation conditions are suitable for infiltration/filtration of the WQCV for a typical site, and the permittee accepts such study as applicable within its MS4 boundaries.

MISC items:

- A Final Drainage Report is required from a Colorado Professional Engineer.

- Drainage basin and bridge fees are due at final plat recordation and are based on the impervious acreage. A 25% fee reduction applies to large lots (2.5 acres or greater). The reduction only applies to the basin fee. The Drainage Report will provide the fee calculation and amount.

- This property is in Zone X, Area of Minimal Flood Hazard, based upon review of the FEMA FIRM. LDC 6.3.2

- There is small portion of  Zone A floodplain in the SE corner of the property.

Impacts to Floodway.

- Prohibited Activities. The following are prohibited:(a)Any encroachments, including the deposit of fill, and other development, unless certification by a professional engineer licensed by the state of Colorado is provided demonstrating the encroachments shall not result in any increase in 100-year flood height, 100-year flood discharge or 100-year floodplain width.

(b) Placement of any mobile homes except in an existing Mobile Home Park or existing Mobile Home Subdivision.

(2) Improvements. PPRBD Approval is required for the following:

(a)All substantial improvements shall comply with all applicable flood hazard reduction provisions of the RBC313.18 through RBC313.21 of the Pikes Peak Regional Building Code, as may be amended.

(b) When a proposed floodway improvement will cause an increase in the base flood elevation (BFE), the requirements of the Pikes Peak Regional Building Code must be met as a condition of approval.

Traffic Impacts

Requirements

| ☒ | Traffic Impact Study/Memorandum | ☒ | Road Impact Fee | ☐ | Joint Access Easement |
| --- | --- | --- | --- | --- | --- |
| ☐ | Road Construction Plans | ☒ | Road Impact Fee Acknowledgement Form | ☐ | County/CDOT project coordination required |
| ☐ | Public Improvements | ☒ | Driveway Access Permit | ☐ | Outsourcing of Traffic Impact Study |
| ☐ | Deviation Request | ☐ | Signal Warrant Study | ☐ | New Road Access Permit |

| Adjacent Road Name |  | Ex. ROW (ft) | Surface | PCI Rating |  | 2045 MTCP |  |  | 2065 MTCP |  |  | Required Intersection Spacing (ft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Classification | ROW | Dedication | Classification | ROW | Potential Preservation |  |
| Judge Orr Road |  | 60 | paved |  |  | Major collector |  |  |  |  |  |  |
| Major collector |  |  |  |  |  |  |  |  |  |  |  | 1/4mi |
| Berridge Rd local |  | 60 | gravel |  |  | local |  |  |  |  |  |  |
| Oil Well Rd local |  | 60 | gravel |  |  | local |  |  |  |  |  |  |
|  |  |  | Other (i.e. multi-jurisdictional): |  |  |  |  |  |  |  |  |  |
| Metro District |  |  |  |  | Choose an item. |  |  |  |  |  |  |  |

Traffic Notes:

- Rezone will require a TIS for RVP or other uses unless the exemption can be met

- Letter of intent to discuss ECM exemption B.1.2.D for note requiring an TIS.

- Driveway access permit will be required for any new access points

- Access from local classification roads to east and west

- EPC Road Impact fees will apply.  Paid at time of building permit    https://publicworks.elpasoco.com/road-impact-fees/

ECM B.1.2.D.

No TIS Required. No TIS is required if all of the criteria below are satisfied:

• Vehicular Traffic: (1) Daily vehicle trip-end generation is less than 100 or the peak hour trip generation is less than 10; (2) there are no additional proposed minor or major roadway intersections on major collectors, arterials, or State Highways; (3) the increase in the number of vehicular trips does not exceed the existing trip generation by more than 10 peak hour trips or 100 daily trip ends; (4) the change in the type of traffic to be generated (i.e., the addition of truck traffic) does not adversely affect the traffic currently planned for and accommodated within, and adjacent to, the property; (5) acceptable LOS on the adjacent public roadways, accesses, and intersections will be maintained; (6)no roadway or intersection in the immediate vicinity has a history of safety or accident problems; and (7) there is no change of land use with access to a State Highway.

Traffic:

- A Traffic Impact Study is required, but the requirement may be waived if all criteria under the TIS exemption in ECM B.1.2.D is met. This subdivision is anticipated to quality for the exemption. This needs to be addressed in the Letter of Intent.

Vehicular Traffic: (1) Daily vehicle trip-end generation is less than 100 or the peak hour trip generation is less than 10; (2) there are no additional proposed minor or major roadway intersections on major collectors, arterials, or State Highways; (3) the increase in the number of vehicular trips does not exceed the existing trip generation by more than 10 peak hour trips or 100 daily trip ends; (4) the change in the type of traffic to be generated (i.e., the addition of truck traffic) does not adversely affect the traffic currently planned for and accommodated within, and adjacent to, the property; (5) acceptable LOS on the adjacent public roadways, accesses, and intersections will be maintained; (6)no roadway or intersection in the immediate vicinity has a history of safety or accident problems; and (7) there is no change of land use with access to a State Highway.

- The El Paso County Road Impact Fee program is applicable to this development. Fees are assessed by type of use. This application would be classified under <>. Current fees are <>. Fees are due at building permit. More information about the program can be found at the following link.

  - https://publicworks.elpasoco.com/road-impact-fees/

- Turnaround areas are required for dead-end roads longer than 300 feet. Per ECM 2.3.8, the maximum length for a rural cul-de-sac is 1,600 feet with a 50’ minimum radius bulb. A deviation request will need to be submitted for consideration to the ECM administrator. A letter from the Fire Department stating that they are acceptable with the proposed roadway length and configuration will be required prior to EPC consideration. Intermediate turnaround points may be required by the Fire Department.

- A Declaration of Easement and Joint Roadway Maintenance Agreement for the proposed private roadway serving the subdivision will need to be recorded along with the final plat. If there is an existing maintenance agreement, it will need to be updated with the proposed development included in the agreement. These documents are recorded with EPC Clerk and Recorders Office.

- A County New Road Access Permit is required for any roadways constructed privately or by another jurisdiction that will generate traffic onto a County owned and maintained roadway. This applies to both the proposed roadway for the subdivision and for the construction of Penny’s Peak View.

- El Paso County Driveway Access Permit is required before any access is established from the proposed lots to the proposed roadway. This is due at time of building permit for new construction. The application can be completed online via EDARP: https://epcdevplanreview.com/Applications/Create_Email under Access Permit (standalone).

- Driveway Access Permits will be needed for the new driveways. This permit can be completed on EDARP and the current cost is $185. The Access Permit is due at building permit.  Follow this link and then select Access Permit (standalone): https://epcdevplanreview.com/Applications/Create_Email

  - Waiver is $82.

Road design requirement per CHPT 6 of the LDC

Roads. This Section shall apply to all roads providing access to a planned building area whether or not they are dedicated as public roads.

(a)

Roads Constructed to County Standards. All roads, including private roads and emergency vehicle access roads, shall be designed and constructed according to this Code and the ECM. Emergency vehicle access roads shall, at a minimum, be constructed to the County's gravel road standard if open to the public. Emergency vehicle access roads which are not open to public travel shall meet the non-road access standards

(b)

Roads within 150 Feet of Development. Roads or emergency vehicle lanes shall be provided within 150 feet of all development except single family residential development.

(c)

Two Access Routes Required. Access to a planned building area shall be provided by a minimum of 2 separate routes in accordance with the requirements of this Code and the ECM if the cul-de-sac exceeds the length allowed by the ECM.

(d)

Turnaround Required on Dead-End Roads. Every dead-end road more than 300 feet in length shall be provided with a roadway termination meeting ECM standards.

(e)

Road Grades in Wildland Fire Areas. Within wildland fire areas, road grades steeper than 10 percent may be permitted where the Fire Authority and ECM Administrator recommend that the mitigation measures are adequate and the approval authority approves the mitigation measure.

(3)

Non-Road Access. The following minimum standards shall apply to emergency vehicle lanes, driveways, and parking lot drive lanes serving as emergency vehicle lanes.

(a)

Emergency Access Provided. Access for emergency responders, ingress, egress, and evacuation shall be provided for all buildings.

(b)

Driveways Required. Where any point of a building is greater than 150 feet from a road, a driveway meeting these standards shall be provided to within 150 feet of the furthest point on the building.

(c)

Emergency Vehicle Lanes Required. The Fire Authority may recommend emergency vehicle lanes be provided. Emergency vehicle lanes shall be provided as required by the approval authority.

(d)

Emergency Access Lane Design. An emergency vehicle lane shall be designed and constructed to enable fire-fighting apparatus to maneuver broadside or directly forward within a minimum of 5 feet and a maximum of 25 feet of structures.

(e)

Width of Driveway and Emergency Vehicle Lanes. Where the driveway is greater than 150 feet in length, it shall be not less than 10 feet in unobstructed width. Emergency vehicle lanes providing one-way travel shall be a minimum of 16 feet in width, and fire lanes with two-way travel shall be a minimum of 24 feet in width.

(f)

Vertical Clearance. At least 13 feet 6 inches of vertical clearance shall be provided and maintained over the full width of an emergency vehicle lane or driveway.

(g)

Turns. Required driveways shall be designed, constructed, and maintained to accommodate the turning radius of the largest apparatus typically used to respond to that location. A turn in an emergency vehicle lane shall be constructed with a minimum radius of 25 feet at the inside curb line and a minimum radius of 50 feet at the outside curb line.

(h)

Grades. Road grades steeper than 10 percent may be permitted where the Fire Authority and ECM Administrator recommend that the mitigation measures are adequate and the approval authority approves the mitigation measure.

(i)

Emergency Vehicle Lanes Connecting to Roads. Emergency vehicle lanes connecting to roads shall be provided with curb cuts extending at least 2 feet beyond each edge of the fire lane.

(j)

Turnouts and Turnarounds Required.

(i)

Driveways. Where the required driveway is greater than 300 feet, it shall be provided with turnouts or turnarounds at locations approved by the approval authority with recommendation from the Fire Authority.

(ii)

Turnarounds Required. The fire authority may provide a recommendation regarding turnarounds. Dead-end emergency vehicle lanes in excess of 300 feet in length shall be provided with turnouts and turnarounds as approved by the approval authority. The turnaround at the terminus shall have a minimum radius of 50 feet. The approval authority shall be authorized to approve, as an alternative, a "hammerhead" turnaround to provide emergency vehicles with a three-point turnaround.

(k)

Load Design. Emergency vehicle lanes and required driveways shall be designed, constructed, and maintained to accommodate the load of the largest apparatus typically used to respond to that location.

(l)

Bridges or Drainage Crossings. A bridge or drainage crossing on an emergency vehicle lane or required driveway shall be designed to accommodate the load of the largest apparatus typically used to respond to that location. The load limit shall be clearly posted at the approaches to the bridge.

COUNTY CODES AND CRITERIA:

El Paso County Land Development Code (LDC) (2016): https://planningdevelopment.elpasoco.com/land-development-code/

El Paso County Engineering and Drainage Criteria Manuals (ECM) (2020) & (DCM) Volumes 1 and 2 and Update:

https://planningdevelopment.elpasoco.com/planning-community-development/engineering/#1519834440345-f2ddfd20-0d90

El Paso County 2024 Major Transportation Corridors Plan Update (MTCP): El Paso County 2045 Major Transportation Corridors Plan (MTCP) (2045mtcp.com)

El Paso County Road Impact Fee Program: https://publicworks.elpasoco.com/road-impact-fees/

El Paso County Development Fees: https://planningdevelopment.elpasoco.com/#1515613078895-69552b09-44a4

El Paso County Projects (EDARP):  https://epcdevplanreview.com/Public

El Paso County Municode Library: https://library.municode.com/co/el_paso_county

Planning and Community Development: https://planningdevelopment.elpasoco.com/planning-development-forms/#1643393391537-a0dac52d-1b3b

El Paso County Stormwater Page: https://publicworks.elpasoco.com/stormwater/

Assessor’s Information

Assessor’s Data:  https://property.spatialest.com/co/elpaso/#/  		Assessor’s Map:  https://community.spatialest.com/co/elpaso/#/Property-Search/

Drainage Links:

Mile High Flood District: http://MHFD.org

Detention Pond Compliance Website: https://maperture.digitaldataservices.com/gvh/?viewer=cswdif

State DWR: http://water.state.co.us/SURFACEWATER/DAMSAFETY/Pages/DamSafety.aspx

NOAA Rainfall: https://hdsc.nws.noaa.gov/hdsc/pfds/pfds_map_cont.html?bkmrk=co

Floodplain: https://www.pprbd.org/Download/Floodplain#floodplainHandouts

https://pprbd.maps.arcgis.com/apps/webappviewer/index.html?id=1d9243f3606542159a0a418070b08686

https://msc.fema.gov/portal/advanceSearch

Fountain Creek Watershed: http://www.fountain-crk.org/

Upper Black Squirrel Creek Ground Water Management District Mapping Study: https://upperblacksquirrelcreekwater.com/waterStudies.php

Water Rights:

Colorado Division of Natural Resources: https://dwr.state.co.us/Portal/dwr/AskDWR

State Transportation Links

Colorado Department of Transportation (CDOT) M&S Standards and Specifications, Access Code and Permit:

  - https://www.codot.gov/business/designsupport/standard-plans

  - https://www.codot.gov/business/permits/accesspermits

  - https://www.codot.gov/business/permits/accesspermits/references

Local Governing Authorities

City of Colorado Springs: https://coloradosprings.gov/

City of Fountain: https://www.fountaincolorado.org/

Town of Calhan: http://calhan.co/

Green Mountain Falls: https://gmfco.elpasoco.com/

Town of Monument: http://www.townofmonument.org/

Town of Palmer Lake: http://www.townofpalmerlake.com/

Contact Information

| CDOT R2 Permits Program Manager | (719)562-5540 |
| --- | --- |
| Work in Right of Way Permit | (719)520-6869 |
| Right of Way Vacation | (719)520-6897 |
| Floodplain Administrator | (719)327-2898 |
| EPC Public Health | (719)578-3199 |
| CDPHE Air Quality Division | (303)692-3100 |
| CDPHE Water Quality Division | (303)692-3500 |
| Cherry Creek Basin Water Quality Authority | (303)239-5400 |

Traffic Information

| Traffic Impact Study (ECM Appendix B)* |  |
| --- | --- |
| Full TIS | ADT > 1,000 or Pk Hr > 100 |
| Intermediate TIS | ADT < 1,000 or Pk Hr < 100 |
| Traffic Memo | ADT ≤ 500 or Pk Hr ≤ 50 |
| No TIS | ADT < 100 or Pk Hr < 10 |

*An approved TIS that has been prepared in the last three years may be revised or updated where a proposed access is changed or a change in the proposed action may result in a new trip generation that exceeds the original trip generation estimates. An amendment letter is required.

**If the original TIS is older than three years, an entirely new TIS shall be prepared.

***Any submitted TIS is subject to third party review based on size and complexity, in which additional fees will be assessed to the applicant.

The applicant is responsible for obtaining any necessary approvals for impacts within other jurisdictions. It is recommended that the applicant approach the jurisdiction early in the development process for any additional requirements. If the proposal is impacted by an El Paso County Department of Public Works project, coordination with DPW is required.

Pavement Condition Index

The Pavement Condition Index (PCI) is a numerical rating system that measures the condition of a road's pavement on a scale of 0 to 100. A score of 100 indicates a newly paved road, while 0 is the worst possible condition. The PCI is calculated based on the type and severity of distresses observed on the pavement surface, such as cracking, rutting, scaling, and spalling. Other factors that can affect the PCI score include Pavement age, Climate and precipitation, Traffic loads, and maintenance.

Drainage Report/Plan Information

Drainage Basin Planning Study (DBPS) – May be required with a very large development

Master Development Drainage Plan (MDDP) – Required with phased development greater than 10 acres (may be required with Sketch Plan) (DCM Section 4.2)

Drainage Letter Report – With a Re-plat, Minor Sub or Plot Plan (DCM Section 4.5) where a final drainage report has previously been approved.

Preliminary Drainage Report (PDR) – Required with a Preliminary Plan (DCM Section 4.3)

Final Drainage Report (FDR) – Required with a Final Plat (DCM Section 4.4)

Drainage Master Plan/Floodplain Information

If it is undetermined at this time if an adopted Drainage Basin Planning Study (DBPS) or Master Plan exists in this area, the applicant is responsible for researching the appropriate information.

If the subdivision application impacts the FEMA floodplain, a FEMA Letter of Map Revision may be required. If the site is in or near a floodplain, contact the Regional Floodplain Administrator for allowed floodplain uses and procedural requirements. It is the Applicant’s responsibility to research the effects and implications of developing in or near a floodplain, including limited uses, floodplain development permits, geotechnical, wetland and wildlife studies, structural requirements, flood insurance and potential future floodplain mapping updates. FEMA's FIRM maps may be out of date or at a low level of accuracy.

If the site lies within the Cherry Creek Basin Water Quality Authority area and is not rural density, the applicant should discuss any concerns related to the development with the CCBWQA’s consultant.

If the site lies within the Fountain Creek watershed, the District has authority over the floodplain impacts and is advisory to the County on uses outside of the floodplain. The applicant should discuss any concerns related to the development with District staff.

Detention/Water Quality PCM & Downstream Conveyance Information

Per ECM Chapter 3.2.8.B: “The proposed project or developed land use shall not change historical runoff values, cause downstream damage, or adversely impact adjacent properties.”

The project must provide an acceptable method of storm drainage conveyance and may be required to construct (or contribute an equitable share to the construction of) a storm conveyance or collector system.

If the site conveys storm drainage flows through or across an adjacent private property, then the applicant is responsible for obtaining off-site drainage easements in accordance with the Engineering Criteria Manual Section 3.3.3.K. Any offsite easements necessary for the development shall be recorded prior to County Plan approvals.

If on-site facilities are required, the applicant will be required to provide access and drainage easements in accordance with Section 11.2.2 of the Drainage Criteria Manual.

If a water quality and/or detention facility is applicable, some or all of the following documents may be required: Operation and Maintenance (O&M) Manual, Private Detention Basin / Stormwater Quality BMP (ie: PCM) Maintenance Agreement, and Stormwater Detention and Infiltration (SDI) Form.

For post-construction water quality PCM requirements, see Appendix I of the Engineering Criteria Manual. If/when the site disturbs >1ac of non-excluded areas, a stormwater quality treatment facility (ie: PCM) will be needed for the runoff from the non-excluded areas. Exclusions can be found in our PCM Applicability Form and ECM Appendix I.7.1.

State Engineer’s requirements regarding dams and water rights apply. Jurisdictional/Non-Jurisdictional dam construction forms are typically required for detention facilities. If applicable, provide proof of submittal of the State’s “Notice of Intent to Construct a Non-Jurisdictional Water Impoundment Structure” form.

Grading, Erosion, and Sediment Control Information

The Grading and Erosion Control (GEC) Plan must be prepared in accordance with Drainage Criteria Manual Vol. 2, Chapter 3 and the County’s GEC Checklist.

An Erosion and Stormwater Quality Control Permit (ESQCP) and Permanent Control Measure are required for the following conditions per ECM Appendix I.4:

- Construction activities that result in land disturbance of greater than or equal (≥) to one acre, or

- Construction activities that result in less than (<) one acre if the activity is part of a larger “Common Plan of Development or Sale” (ECM Chapter 1.22) that would disturb one acre or more of ground surface

An ESQCP may be required as determined by the ECM Administrator for the following conditions per ECM Appendix I.4:

- Construction activities that are located in or adjacent to a Sensitive Area, or

- Cut and/or fill greater than (>) 500 cubic yards of soil

See the three ESQCP exclusions listed in ECM Chapter 5.6.3.

An application for an ESQCP shall be accompanied by the following:

| - Drainage Report - ESQCP Form - Financial Surety - GEC Plan and GEC Checklist | - PCM Applicability Form - Stormwater Management Plan (SWMP) and SWMP Checklist - O&M Manual (for sites requiring a PCM) - PCM Maintenance Agreement (for sites requiring a PCM |
| --- | --- |

A Builder’s Erosion and Stormwater Quality Control Permit (BESQCP) is required for residential building lots with < 1 acre of disturbed areas and does not meet any of the criteria for an ESQCP listed above.

Construction activities that disturb >1ac of soil are required by the Environmental Protection Agency (EPA) to obtain a Construction Stormwater Permit through the Colorado Department of Public Health & Environment (CDPHE) Water Quality Control Divisions. For projects with soil disturbances >1ac but <25ac, a County Construction Activity Permit is required, contact El Paso County’s Public Health Department for further information regarding the permit. All land development activities greater than twenty-five (25) acres or with construction duration longer than six (6) months must obtain an Air Pollution Emission Notice and Emissions Permit through CDPHE’s Air Quality Control Division.

Deviations

All engineering designs and studies shall be performed in conformance with adopted codes, standards and criteria. Any deviations are to be formally identified and requested in writing, with justification provided per ECM Section 5.8. All deviation requests must be approved by the ECM Administrator prior to submitting the application for review, or delays in the review and additional fees may result. The Applicant must submit adequate justification for consideration of the request(s).

Public Improvements Information

If public improvements are required, the applicant will be required to enter into a Subdivision Improvement Agreement (SIA) or a development agreement (if the project is not a subdivision) with the County. Refer to the El Paso County Land Development Code for information on the SIA and the ECM for the required format of the associated Financial Assurance Estimate. All forms are available online.

Construction drawings for the required public improvements must be reviewed and approved by PCD and the County Engineer. All Construction drawing submittals shall adhere to the criteria set forth in the ECM. A construction plan review fee will be assessed when the plans are submitted for review.

Geotechnical reports for earthwork and pavement designs must be reviewed and approved.  All submittals must adhere to the criteria set forth in the ECM.

Any work within the Right of Way will require a permit from El Paso County Department of Public Works.

Any Fire cistern(s) and mailbox kiosks need to be shown on the construction drawings.

Metro District Fees

| Woodmen Road Metro District | Central Marksheffel Metro District | Lorson Ranch Metro District | Constitution Heights Metro District |
| --- | --- | --- | --- |
| - Property in district pays no fee - After December 31, 2020 no property in the Amended Service Area shall be permitted to join the WRMD, and shall participate fully in the Fee Program. - As of January 1, 2021 and until either bonds retire or December 31, 2027, whichever occurs sooner, El Paso County must pay a portion of the Road Impact Fee it collects from property in the Amended Service Area that receives final plat approval to WRMD. | - Property in district pays no fee - District granted $2.9 million in Fee Program Credit - Fee Program will pay annual credit reimbursements to District, or can sell credits | - Property in district pays full fee - District granted $4.1 million in credit - Credits used toward future development, can apply for reimbursement, can sell credits | - Property in district pays full fee - District granted $3.1 million in credit - Used toward future development, can sell credits, NO reimbursement - County will collect building permit fees for reimbursement of PPRTA improvements to Marksheffel Road from property within District and some property outside District – Credit given toward Impact Fee for amount of building permit fee paid. |

*Properties within the Meridian Crossing subdivision are not subject to the county wide road impact fee.