EXHIBIT

JUDGE ORR ROAD COMMERCIAL PARK

CS ZONING

PROPERTY DESCRIPTION

A PARCEL OF LAND BEING A PORTION OF THE SOUTHEAST QUARTER OF SECTION 32 AND THE SOUTHWEST QUARTER OF SECTION 33, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6TH PRINCIPAL MERIDIAN, COUNTY OF EL PASO, STATE OF COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE SOUTH LINE OF THE SOUTHEAST QUARTER OF SECTION 32, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6TH PRINCIPAL MERIDIAN, BEING MONUMENTED AT THE SOUTH QUARTER CORNER BY A 3.25" ALUMINUM CAP STAMPED "PLS 22573" AND AT THE SOUTHEAST CORNER BY A 3.25" ALUMINUM CAP STAMPED "PLS 17496", BEARING N89°56'47"E AS REFERENCED TO COLORADO STATE PLANE CENTRAL ZONE.

BEGINNING AT THE SOUTHEAST CORNER OF SECTION 32, TOWNSHIP 12 SOUTH, RANGE 64 WEST OF THE 6TH PRINCIPAL MERIDIAN:

THENCE ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF SAID SECTION 32, S89°56'47"W A DISTANCE OF 1904.48 FEET, TO A POINT ON THE EASTERLY RIGHT-OF-WAY LINE OF HIGHWAY 24;

THENCE ON SAID EASTERLY RIGHT-OF-WAY LINE, THE FOLLOWING THREE (3) COURSES:

- N45°55'29"E A DISTANCE OF 121.27 FEET, TO A POINT OF CURVE;

- ON THE ARC OF A CURVE TO THE LEFT, HAVING A RADIUS OF 5800.00 FEET, A CENTRAL ANGLE OF 01°30'00" AND AN ARC LENGTH OF 151.84 FEET, TO A POINT OF TANGENT;

- N44°25'29"E A DISTANCE OF 1577.02 FEET;

THENCE DEPARTING SAID EASTERLY RIGHT-OF-WAY LINE, N89°57'40"E A DISTANCE OF 592.85 FEET, TO A POINT ON THE EAST LINE OF THE SOUTHEAST QUARTER OF SAID SECTION 32;

THENCE DEPARTING SAID EAST LINE, THE FOLLOWING FOUR (4) COURSES:

- N89°59'19"E A DISTANCE OF 159.41 FEET;

- S00°30'12"E A DISTANCE OF 905.50 FEET;

- S07°12'45"W A DISTANCE OF 89.38 FEET;

- S00°30'12"E A DISTANCE OF 322.05 FEET, TO THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 33;

THENCE ON SAID SOUTH LINE, S89°56'50"W A DISTANCE OF 146.03 FEET, TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 1,844,015 SQUARE FEET OR 42.3328 ACRES.

EXHIBIT ATTACHED AND BY THIS REFERENCE MADE A PART HEREOF.

PROPERTY DESCRIPTION STATEMENT

I, DEREK LEE VAGIAS, A PROFESSIONAL LAND SURVEYOR LICENSED IN THE STATE OF COLORADO, DO HEREBY STATE THAT THE ABOVE PROPERTY DESCRIPTION AND ATTACHED EXHIBIT WERE PREPARED UNDER MY RESPONSIBLE CHARGE, AND ON THE BASIS OF MY KNOWLEDGE, INFORMATION AND BELIEF, ARE CORRECT.

DEREK LEE VAGIAS, PROFESSIONAL LAND SURVEYOR

COLORADO NO. 38578

FOR AND ON BEHALF OF JR ENGINEERING, LLC