LEGAL DESCRIPTION:   ZONE CHANGE

A TRACT OF LAND BEING A PORTION OF THE NORTHEAST QUARTER OF SECTION 2, TOWNSHIP 15 SOUTH, RANGE 66 WEST OF THE 6TH P.M., EL PASO COUNTY, STATE OF COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS:

THE NORTH LINE OF NORTHEAST QUARTER OF SECTION 2, TOWNSHIP 15

SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO

COUNTY, COLORADO, BEING MONUMENTED AT THE WESTERLY END BY

TWO REFERENCE MONUMENTS BEING 3-1/2” ALUMINUM CAPS STAMPED

“PLS 24313” AT GRADE AND AT THE EASTERLY END BY A 3-1/2” ALUMINUM

CAP STAMPED ”URS  WC 6’ T14S R66W S35 S36 S2 S1 T15S R66W 2012

PLS 24313” AT GRADE, ASSUMED TO BEAR N89°33'12”E A DISTANCE OF

2,644.67 FEET.

COMMENCING AT THE SOUTHWEST CORNER OF HIGHWAY ACQUISITION AS DESCRIBED IN THE SPECIAL WARRANTY DEED RECORDED UNDER RECEPTION NUMBER 20907404, RECORDS OF EL PASO COUNTY, COLORADO, SAID POINT BEING THE POINT OF BEGINNING; THENCE ON THE BOUNDARY LINE OF SAID WARRANTY DEED THE FOLLOWING TEN (10) COURSES:

- N89°33’12”E A DISTANCE OF 329.56 FEET;

- S84°48’16”E A DISTANCE OF 303.34 FEET;

- N89°46’53”E A DISTANCE OF 301.23 FEET;

- S86°32’43”E A DISTANCE OF 172.68 FEET

- N89°35’07”E A DISTANCE OF 307.71 FEET;

- S29°00’54”E A DISTANCE OF 61.18 FEET;

- S00°41’01”E A DISTANCE OF 90.85 FEET;

- S02°35’52”E A DISTANCE OF 390.19

- S00°46’16”E A DISTANCE OF 213.28 FEET;

- N89°36’17”E A DISTANCE OF 7.96 FEET TO THE WESTERLY RIGHT-OF-WAY OF HANCOCK EXPRESSWAY AS RECORDED IN BOOK 2005, AT PAGE 753;

THENCE S00°46’16”E ON SAID WESTERLY RIGHT-OF-WAY LINE A DISTANCE OF 641.21 FEET; THENCE S89°13’44”W 1,379.62 FEET; THENCE S00°47’31”E A DISTANCE OF 137.31 FEET; THENCE S57°19’44”E A DISTANCE OF 397.59 FEET TO THE MOST NORTHWEST CORNER OF CLEARVIEW DRIVE AS PLATTED IN, CLEAR VIEW INDUSTRIAL PARK FILING NO. 1, RECORDED UNDER PLAT BOOK N-2 AT PAGE 19; THENCE S32°40’16”W ON SAID RIGHT-OF WAY LINE A DISTANCE OF 80.00 FEET; THENCE N57°19’44”W A DISTANCE OF 525.11 FEET; THENCE N32°40’37”E A DISTANCE OF 80.00 FEET; THENCE S85°08’21”E A DISTANCE OF 26.56 FEET; THENCE N00°50’57”W A DISTANCE OF 1,509.86 FEET TO THE POINT OF BEGINNING.

EXCEPTING OUT THE TRACT OF LAND AS DESCRIBED IN WARRANTY DEED RECORDED UNDER RECEPTION NUMBER 225103087.

CONTAINING A CALCULATED TOTAL NET AREA OF 2,091,962 FEET OR 48.025 ACRES.