## LEGAL DESCRIPTION BROADMOOR GLEN ADDITION NO. 1 ANNEXATION

A PARCEL OF LAND BEING PARCEL B (TRACT 101E-2) OF THE RULE AND ORDER RECORDED OCTOBER 22, 2001 UNDER RECEPTION NUMBER 201152953 OF THE OFFICIAL RECORDS OF THE EL PASO COUNTY, COLORADO CLERK AND RECORDER, BEING LOCATED IN A PORTION OF THE NORTH HALF OF THE NORTHEAST QUARTER (N1/2 NE1/4) OF SECTION 18, TOWNSHIP 15 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

COMMENCING AT A NO. 6 REBAR WITH A 2-1/2” ALUMINUM CAP STAMPED BY PLS 19586 AT THE SOUTHEAST CORNER OF SAID N1/2 NE1/4, FROM WHICH A NO. 5 REBAR WITH 1-1/2” ALUMINUM CAP STAMPED BY LS 1808 AT THE SOUTHWEST CORNER OF SAID N1/2 NE1/4 BEARS NORTH 88°59'07" WEST A DISTANCE OF 2641.06 FEET AND IS THE BASIS OF BEARINGS USED HEREIN;

THENCE NORTH 88°59'07" WEST ON THE SOUTH LINE OF SAID N1/2 NE1/4, A DISTANCE OF 1,101.73 FEET TO THE SOUTHEAST CORNER OF SAID PARCEL B AND BEING THE POINT OF BEGINNING;

THENCE CONTINUING NORTH 88°59'07" WEST ON SAID SOUTH LINE, A DISTANCE OF 201.27 FEET TO A SOUTHWEST CORNER OF SAID PARCEL B;

THENCE ON THE WESTERLY, SOUTHWESTERLY, SOUTHERLY, SOUTHEASTERLY AND EASTERLY LINES OF SAID PARCEL B THE FOLLOWING SIX COURSES:

- THENCE NORTH 05°25'12" WEST A DISTANCE OF 229.41 FEET TO A POINT OF CURVE;

- THENCE ON A CURVE TO THE LEFT THROUGH A CENTRAL ANGLE OF 52°00'00", HAVING A RADIUS OF 218.31 FEET FOR AN ARC DISTANCE OF 198.13 FEET;

- THENCE NORTH 57°25'12" WEST A DISTANCE OF 469.78 FEET TO A POINT OF CURVE;

- THENCE ON A CURVE TO THE LEFT THROUGH A CENTRAL ANGLE OF 133°55'00", HAVING A RADIUS OF 99.99 FEET FOR AN ARC DISTANCE OF 233.71 FEET;

- THENCE SOUTH 11°20'12" EAST A DISTANCE OF 311.75 FEET TO A POINT OF CURVE;

- THENCE ON A CURVE TO THE RIGHT THROUGH A CENTRAL ANGLE OF 31°09'17", HAVING A RADIUS OF 418.31 FEET FOR AN ARC DISTANCE OF 227.46 FEET TO THE SOUTH LINE OF SAID N1/2 NE1/4;

THENCE NORTH 88°59'07" WEST ON SAID SOUTH LINE, A DISTANCE OF 224.29 FEET TO A POINT ON A CURVE, FROM WHICH A RADIAL LINE BEARS NORTH 50°50'37" WEST;

THENCE ON THE WESTERLY, NORTHWESTERLY, NORTHERLY, NORTHEASTERLY AND EASTERLY LINES OF SAID PARCEL B FOLLOWING SIX COURSES:

- THENCE ON A CURVE TO THE LEFT THROUGH A CENTRAL ANGLE OF 50°29'35", HAVING A RADIUS OF 218.31 FEET FOR AN ARC DISTANCE OF 192.39 FEET;

- THENCE NORTH 11°20'12" WEST A DISTANCE OF 311.75 FEET TO A POINT OF CURVE;

- THENCE ON A CURVE TO THE RIGHT THROUGH A CENTRAL ANGLE OF 133°55'00", HAVING A RADIUS OF 299.99 FEET FOR AN ARC DISTANCE OF 701.16 FEET;

- THENCE SOUTH 57°25'12" EAST A DISTANCE OF 469.78 FEET TO A POINT OF CURVE;

- THENCE ON A CURVE TO THE RIGHT THROUGH A CENTRAL ANGLE OF 52°00'00", HAVING A RADIUS OF 418.31 FEET FOR AN ARC DISTANCE OF 379.65 FEET;

- THENCE SOUTH 05°25'12" EAST A DISTANCE OF 251.97 FEET TO THE POINT OF BEGINNING.

CONTAINING 9.123 ACRES OF LAND, MORE OR LESS.

JOHN L. BAILEY, PLS 19586

PREPARED FOR AND ON BEHALF OF

CLASSIC CONSULTING ENGINEERS AND SURVEYORS