1/15/2026	Site Visit		Did a Rebar inspection they used a #4 rebar and they had all there ties in the right spots as well as the concrete chairs to hold the rebar up a little. Everything looked great they will be pouring at 3.

Structure being poured: Inlet

Method for pour (machine, handset, etc.): Handset

Slump: 6.5

Mix temperature: 60

Air content: 4.5

Water/Admixtures added (if any): a half bag of super air

Total yards scheduled:20 yrds

Ambient temperature: 50

Weather conditions: Cloudy

Time of pour: 3.00PM

Batch time:

Ticket number

Contractor/Company used: Entech Tester Burnco Failed

Structure being poured: Inlet

Method for pour (machine, handset, etc.): Handset

Slump: 6

Mix temperature: 60

Air content: 6

Water/Admixtures added (if any):

Total yards scheduled:20 yrds

Ambient temperature: 50

Weather conditions: Cloudy

Time of pour: 3.00PM

Batch time:

Ticket number

Contractor/Company used: Entech Tester Burnco Passed have to test 2nd truck

Structure being poured: Inlet

Method for pour (machine, handset, etc.): Handset

Slump: 5.5

Mix temperature: 63

Air content: 4.5

Water/Admixtures added (if any): a half bag of super air

Total yards scheduled:20 yrds

Ambient temperature: 50

Weather conditions: Cloudy

Time of pour: 3.00PM

Batch time:

Ticket number

Contractor/Company used: Entech Tester Burnco Failed this was the 2nd truck

Structure being poured: Inlet

Method for pour (machine, handset, etc.): Handset

Slump: 6

Mix temperature: 65

Air content: 7

Water/Admixtures added (if any): a half bag of super air

Total yards scheduled:20 yrds

Ambient temperature: 50

Weather conditions: Cloudy

Time of pour: 3.00PM

Batch time:

Ticket number

Contractor/Company used: Entech Tester Burnco Passed

A Rivera	Edit | Delete

1/7/2026	Site Visit		Stopped by in the afternoon and they had 3 of the inlets ready for the rebar inspection. they had formed all the bottoms Rebar looked great used a number 4 rebar and made sure they had all there ties.	A Rivera	Edit | Delete

1/6/2026	Site Visit		I got a call from Duke and said they they was all done with installing RCP Took a look at that and all looked great. They also started to form up the inlet boxes and they will be needing Rebar inspections.	A Rivera	Edit | Delete

12/30/2025	Site Visit		Duke had let me know that they was done and i could come and look at it. They did a great job with the grout. It all looked good and they was ready to back fill after the grout had sat for a while. and in the mean time they was going to be laying more RCP.	A Rivera	Edit | Delete

12/29/2025	Site Visit		Duke called and asked if i can come and look at some more RCP. Everything looked great there was a couple that i asked that they grout just because it had a gap in between the two RCP. I was not a big gap but i had they grout it just to be on the safe side. Duke said that he would do that tomorrow before they back filled and asked if i wanted to see and i told him yes i would please.	A Rivera	Edit | Delete

12/22/2025	Site Visit		Duke called for another inspection on more RCP they had put in. Again looked at flow and made sure they had no gaps in the RCP. Everything is looking great and they are just moving along.	A Rivera	Edit | Delete

12/18/2025	Site Visit		Duke with Chiddix called and said they had about 5 Sticks of RCP laid in the ground and was ready for it to be looked at. I did an inspection looked to make sure there was no openings where each one connected and made sure they had the tracer wire on top of the RCP. Also checked the flow and it was sitting at about 1 to 1.5 percent flow.	A Rivera	Edit | Delete

12/16/2025	Site Visit		They had more RCP dropped off so i took a look at the rest of it and all looked great. they are ready to start installing RCP.	A Rivera	Edit | Delete

12/12/2025	Site Visit		Did a RCP inspection. Had about 8 pipes out there to look at. Everything look great.	A Rivera	Edit | Delete

9/3/2025	Site Visit		No workers on site. No inspections needed at this time. Job in good condition temps are 78 degrees.	A Rivera	Edit | Delete