LEGAL DESCRIPTION

A PARCEL OF LAND LOCATED IN THE EAST HALF OF SECTION 9, TOWNSHIP 15 SOUTH, RANGE 65 WEST OF THE SIXTH PRINCIPAL MERIDIAN, CITY OF COLORADO SPRINGS, EL PASO COUNTY, COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS WITH BEARINGS REFERENCED TO THE NORTH-SOUTH CENTERLINE OF SAID SECTION 9, MONUMENTED AT EACH END BY A 6/8 INCH REBAR WITH A 3-1/4 INCH ALUMINUM CAP STAMPED “PLS 10377” FOUND FLUSH WITH GRADE AND DETERMINED BY GPS OBSERVATION TO BEAR NORTH 00°19’23” WEST, A DISTANCE OF 5252.20 FEET.

COMMENCING AT THE SOUTH QUARTER CORNER OF SAID SECTION 9;

THENCE NORTH 00°19’23” WEST, ON SAID NORTH-SOUTH CENTERLINE OF SECTION 9, A DISTANCE OF 3638.44 FEET TO THE SOUTHERLY RIGHT OF WAY OF BRADLEY ROAD (FORMERLY KNOWN AS NEW DRENNAN ROAD) AS DESCRIBED IN A DOCUMENT RECORDED JANUARY 26, 1987 IN THE OFFICE OF THE EL PASO COUNTY CLERK AND RECORDER IN BOOK 5307 AT PAGE 1472 OF SAID EL PASO COUNTY RECORDS;

THENCE NORTH 89°30’06” EAST, ON SAID SOUTHERLY RIGHT OF WAY LINE, A DISTANCE OF 475.70 FEET TO A POINT ON THE EASTERLY LINE OF THAT CERTAIN EASEMENT DESCRIBED IN A DOCUMENT RECORDED JUNE 6, 2002 UNDER RECEPTION NUMBER 202091474 OF SAID EL PASO COUNTY RECORDS, SAID POINT ALSO BEING THE POINT OF BEGINNING;

THENCE EASTERLY, ON SAID SOUTHERLY RIGHT OF WAY, THE FOLLOWING (3) COURSES;

- THENCE CONTINUE NORTH 89°30'06" EAST A DISTANCE OF 539.68 FEET TO A NON-TANGENT CURVE, HAVING A RADIUS OF 150.00 FEET, WHOSE CENTER BEARS SOUTH 00°30'06" EAST;

- THENCE EASTERLY, ON SAID CURVE, THROUGH A CENTRAL ANGLE OF 21°02'34", AN ARC DISTANCE OF 55.09 FEET;

- THENCE NORTH 89°30'06" EAST, A DISTANCE OF 127.69 FEET TO THE WESTERLY RIGHT OF WAY OF BRADLEY LANDING BOULEVARD AS DEDICATED BY THE PLAT OF BRADLEY HEIGHTS ROADS FILING NO. 1 RECORDED NOVEMBER 16, 2023 UNDER RECEPTION NUMBER 223715224 OF SAID EL PASO COUNTY RECORDS;

THENCE SOUTHERLY, ON SAID RIGHT OF WAY, THE FOLLOWING (3) COURSES;

- THENCE SOUTH 00°29'54" EAST A DISTANCE OF 44.86 FEET TO A TANGENT CURVE, HAVING A RADIUS OF 718.50 FEET, WHOSE CENTER BEARS WESTERLY;

- THENCE SOUTHERLY, ON SAID CURVE, THROUGH A CENTRAL ANGLE OF 44°03'28", AN ARC DISTANCE OF 552.49 FEET TO A 802.00 FOOT REVERSE CURVE;

- THENCE SOUTHERLY, ON SAID CURVE, THROUGH A CENTRAL ANGLE OF 58°20'37", AN ARC DISTANCE OF 816.67 FEET TO THE NORTHWEST CORNER OF BRADLEY LANDING BOULEVARD AS DEDICATED BY THE PLAT OF BRADLEY HEIGHTS ROADS FILING NO. 2 RECORDED JANUARY 8, 2024 UNDER RECEPTION NUMBER 224715247 OF SAID EL PASO COUNTY RECORDS;

THENCE CONTINUE SOUTHERLY, ON SAID WESTERLY RIGHT OF WAY AND SAID 802.00 RADIUS CURVE, THROUGH A CENTRAL ANGLE OF 05°08'47", AN ARC DISTANCE OF 72.04 FEET TO THE EXTERIOR BOUNDARY OF THAT CERTAIN PARCEL OF LAND DESCRIBED IN A DOCUMENT RECORDED DECEMBER 22, 2021 UNDER RECEPTION NUMBER 221232528 OF SAID EL PASO COUNTY RECORDS;

THENCE SOUTH 70°03'33" WEST, ON THE EXTERIOR OF SAID PARCEL OF LAND RECORDED UNDER RECEPTION NUMBER 221232528, A DISTANCE OF 12.70 FEET TO THE PREVIOUSLY CITED EASTERLY LINE OF SAID EASEMENT DESCRIBED IN A DOCUMENT RECORDED JUNE 6, 2002 UNDER RECEPTION NUMBER 202091474 OF SAID EL PASO COUNTY RECORDS;

THENCE NORTHWESTERLY, ON THE EXTERIOR OF SAID PARCEL OF LAND RECORDED UNDER RECEPTION NUMBER 221232528 BEING COMMON WITH THE SAID EASTERLY LINE OF SAID EASEMENT RECORDED UNDER RECEPTION NUMBER 202091474 THE FOLLOWING 2 COURSES;

THENCE NORTHWESTERLY, ON SAID EASTERLY LINE, THE FOLLOWING (2) COURSES;

- THENCE NORTH 19°56'43" WEST A DISTANCE OF 292.74 FEET;

- THENCE NORTH 12°13'03" WEST A DISTANCE OF 1,130.77 FEET TO THE POINT OF BEGINNING.

THE PARCEL, AS DESCRIBED ABOVE, PRODUCES A CALCULATED AREA OF 449,577 SQUARE FEET (10.32087 ACRES), MORE OR LESS, AND IS DEPICTED ON THE ATTACHED GRAPHICAL EXHIBIT FOR REFERENCE.

ROBERT L. MEADOWS JR., PLS 34977

PREPARED FOR AND ON BEHALF OF

CLASSIC CONSULTING ENGINEERS AND SURVEYORS