EA File No. EA25103 Old Denver Storage

Engineering Meeting Notes

| EA Meeting Date/Time: | Monday, 9/29/2025 10:00 |
| --- | --- |
| Parcel Number: | 7126004010 |
| Address./Plat No./Acre: | 16050 OLD DENVER RD Monument 80132 // NOT PLATTED // 8.07ac |
| Project Manager/Phone: | Kylie Bagley (719) 520-6323 |
| Development Services Engineer/Phone: | Ed Schoenheit (719) 520-6813 |
| Stormwater Engineer/Phone | Zachary Jannusch (719) 500-3170 |
| Application/Land Use Type: | REPLAT, SUB, SDP |

## IMPORTANT NOTE:

These Engineering meeting notes are based on the information provided by the Applicant and reasonable preliminary research.  The County requirements and policies in effect at the time of the meeting may change prior to the project submittal date, and the requirements and policies in effect at the date of submittal shall apply.  Based on the applicant-provided information and preliminary research, these notes are the best estimate of the requirements expected to be met by the Applicant.  The actual requirements may change based on project revisions, new information, or constraints that were unavailable or overlooked at the time of the Early Assistance meeting.  It is solely the Applicant’s responsibility to research and be familiar with state and federal laws and permitting requirements; the County’s regulations, codes and criteria; the requirements of other applicable local, state or federal agencies; and any governing documents that apply to the project, including previous drainage and transportation studies and previous land use approvals and conditions. Reference links are provided at the end of this document.

REQUEST

Subdividing the 8 acre property into 2 or 3 lots and developing the northwest portion 2.5 to 3 acres into an outdoor RV & boat storage facility on a recycled asphalt parking lot out of and above the 100 year floodplain. Will only be looking to develop the 3 acre northwest portion of the 8 acre property. Single family home on the 3 acre southwest portion of the property will remain as a non conforming use CS235 CS Rezone EA2349

Drainage Impacts

Requirements

| Documents |  | Design Elements |  |
| --- | --- | --- | --- |
| ☐ | Drainage Basin Planning Study (DBPS) | ☒ | Full Spectrum Detention |
| ☐ | Master Development Drainage Plan (MDDP) | ☒ | Permanent Water Quality |
| ☒ | Preliminary/Final Drainage Report/Letter | ☒ | Downstream Conveyance Analysis (to include possible offsite improvements) |
| ☒ | Financial Assurance Estimate (FAE) Form | ☒ | Floodplain Impacts |
| ☐ | Builder’s Erosion and Stormwater Quality Control Permit (BESQCP) | ☐ | Public Improvements |
| ☒ | Erosion and Stormwater Quality Control Permit (ESQCP) |  |  |
| ☒ | Grading & Erosion Control (GEC) Plan & GEC Checklist | Miscellaneous |  |
| ☒ | Stormwater Management Plan (SWMP) & SWMP Checklist | ☒ | Construction Activity Permit/ CDPHE Discharge Permit/CDPHE APEN Fee |
| ☒ | PCM Applicability Form | ☐ | Special Districts/Water Quality Authority Areas |
| ☒ | PCM Maintenance Agreement and O&M Manual |  |  |
| ☒ | MHFD-Detention Basin Design Workbook or SDI Sheet |  |  |
| ☐ | Pre-Subdivision Site Grading Form |  |  |
| ☐ | Deviation Request |  |  |

| Basin Name/Studied | Basin Fee* | Bridge Fee* |
| --- | --- | --- |
| TEACHOUT CREEK | $7337 | $1103 |

* Per impervious acre, collected prior to recordation of Final Plat.

- A 25% fee reduction applies for Low Density Lots (2.5 acres and greater lot sizes).

Drainage Notes:

- A final drainage report (FDR) will be required Drainage letter prepared by a professional engineer licensed in the state of Colorado.

- The flood map for the selected area is number 08041C0278G, effective on12/7/2018. Zone AE Floodplain is present on the parcel.

- For a subdivision and plat A GEOTECH report will be required with borings on new lots. See ECM Appdx C for GEOTECH report standards.

- Drainage basin and bridge fees are applicable for a plat and paid at time of Final plat recording.  Commercial impervious value is 95%. Drainage report must provide analysis for impervious value to be used for the development.

- Final Plat Application: Final Drainage Report, ESQCP, FAE, SWMP & Checklist, Grading and Erosion Control Plan & Checklist, SDI Worksheet, Pond Maintenance Agreement, Pond O&M, Construction Drawings

- Drainage letter must analyze culverts that carry flow and all offsite flows

- An EPC Stormwater Permit (ESQCP) will be needed if/when one of the following criteria are triggered:

  - Develop the site and disturb >1ac of soil, or:

  - <1ac of soil disturbance but the site is part of a larger common plan of development, or:

  - Cut and/or fill >500 cubic yards of soil, or:

  - The site is located in or adjacent to a sensitive area.

- If/when the site disturbs >1ac of soil, a stormwater quality treatment facility (PCM) will be needed for the runoff from the non-excluded areas. Exclusions can be found in our PCM Applicability Form and ECM Appendix I.7

- Land disturbance of 1 acre or greater or 500 cy triggers:

  - ESQCP

  - GEC plans, GEC checklist, SWMP, BMP agreement, FAE, PBMP app, and SWMP checklist

  - Full spectrum detention and water quality, SDI sheet

Traffic Impacts

Requirements

| ☒ | Traffic Impact Study/Memorandum | ☒ | Road Impact Fee | ☐ | Joint Access Easement |
| --- | --- | --- | --- | --- | --- |
| ☐ | Road Construction Plans | ☒ | Road Impact Fee Acknowledgement Form | ☐ | County/CDOT project coordination required |
| ☐ | Public Improvements | ☒ | Driveway Access Permit | ☐ | Outsourcing of Traffic Impact Study |
| ☐ | Deviation Request | ☐ | Signal Warrant Study | ☐ | New Road Access Permit |

| Adjacent Road Name |  | Ex. ROW (ft) | Surface | PCI Rating |  | 2045 MTCP |  |  | 2065 MTCP |  |  | Required Intersection Spacing (ft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Classification | ROW | Dedication | Classification | ROW | Potential Preservation |  |
| Old Denver Hwy |  | 95ft | paved |  |  | Major Collector | 90ft |  |  |  |  | 660ft |
|  |  |  | Other (i.e. multi-jurisdictional): |  |  |  |  |  |  |  |  |  |
| Metro District |  |  |  |  | Choose an item. |  |  |  |  |  |  |  |

Traffic Notes:

- BoCC Resolution 24-187  CS Rezone

1.	A TIS will be required completed by a CO licensed traffic engineer.

2.	TIS must address fullest and best use within the study with all planned used and cover the entire parcel.

3.	Road Impact fee will be applicable.  https://publicworks.elpasoco.com/road-impact-fees/

4.	All public improvements and road impact fees will be identified in the study. Study to address adjacent parcel access and traffic generation.

5.	An access easement should be obtained for use of the existing MVEA driveway.

6.	A driveway access permit will be required for all access points.

7.	Paving is required for the driveway access point for first 20ft for a driveway entrance and 50ft for a commercial entrance.

8.	All connections to roadway must be at 90 degrees

No TIS Required. No TIS is required if all of the criteria below are satisfied:

• Vehicular Traffic: (1) Daily vehicle trip-end generation is less than 100 or the peak hour trip generation is less than 10; (2) there are no additional proposed minor or major roadway intersections on major collectors, arterials, or State Highways; (3) the increase in the number of vehicular trips does not exceed the existing trip generation by more than 10 peak hour trips or 100 daily trip ends; (4) the change in the type of traffic to be generated (i.e., the addition of truck traffic) does not adversely affect the traffic currently planned for and accommodated within, and adjacent to, the property; (5) acceptable LOS on the adjacent public roadways, accesses, and intersections will be maintained; (6)no roadway or intersection in the immediate vicinity has a history of safety or accident problems; and (7) there is no change of land use with access to a State Highway.

Rezone BoCC Requirements.

COUNTY CODES AND CRITERIA:

El Paso County Land Development Code (LDC) (2016): https://planningdevelopment.elpasoco.com/land-development-code/

El Paso County Engineering and Drainage Criteria Manuals (ECM) (2020) & (DCM) Volumes 1 and 2 and Update:

https://planningdevelopment.elpasoco.com/planning-community-development/engineering/#1519834440345-f2ddfd20-0d90

El Paso County 2024 Major Transportation Corridors Plan Update (MTCP): El Paso County 2045 Major Transportation Corridors Plan (MTCP) (2045mtcp.com)

El Paso County Road Impact Fee Program: https://publicworks.elpasoco.com/road-impact-fees/

El Paso County Development Fees: https://planningdevelopment.elpasoco.com/#1515613078895-69552b09-44a4

El Paso County Projects (EDARP):  https://epcdevplanreview.com/Public

El Paso County Municode Library: https://library.municode.com/co/el_paso_county

Planning and Community Development: https://planningdevelopment.elpasoco.com/planning-development-forms/#1643393391537-a0dac52d-1b3b

El Paso County Stormwater Page: https://publicworks.elpasoco.com/stormwater/

Assessor’s Information

Assessor’s Data:  https://property.spatialest.com/co/elpaso/#/  		Assessor’s Map:  https://community.spatialest.com/co/elpaso/#/Property-Search/

Drainage Links:

Mile High Flood District: http://MHFD.org

Detention Pond Compliance Website: https://maperture.digitaldataservices.com/gvh/?viewer=cswdif

State DWR: http://water.state.co.us/SURFACEWATER/DAMSAFETY/Pages/DamSafety.aspx

NOAA Rainfall: https://hdsc.nws.noaa.gov/hdsc/pfds/pfds_map_cont.html?bkmrk=co

Floodplain: https://www.pprbd.org/Download/Floodplain#floodplainHandouts

https://pprbd.maps.arcgis.com/apps/webappviewer/index.html?id=1d9243f3606542159a0a418070b08686

https://msc.fema.gov/portal/advanceSearch

Fountain Creek Watershed: http://www.fountain-crk.org/

Upper Black Squirrel Creek Ground Water Management District Mapping Study: https://upperblacksquirrelcreekwater.com/waterStudies.php

Water Rights:

Colorado Division of Natural Resources: https://dwr.state.co.us/Portal/dwr/AskDWR

State Transportation Links

Colorado Department of Transportation (CDOT) M&S Standards and Specifications, Access Code and Permit:

  - https://www.codot.gov/business/designsupport/standard-plans

  - https://www.codot.gov/business/permits/accesspermits

  - https://www.codot.gov/business/permits/accesspermits/references

Local Governing Authorities

City of Colorado Springs: https://coloradosprings.gov/

City of Fountain: https://www.fountaincolorado.org/

Town of Calhan: http://calhan.co/

Green Mountain Falls: https://gmfco.elpasoco.com/

Town of Monument: http://www.townofmonument.org/

Town of Palmer Lake: http://www.townofpalmerlake.com/

Contact Information

| CDOT R2 Permits Program Manager | (719)562-5540 |
| --- | --- |
| Work in Right of Way Permit | (719)520-6869 |
| Right of Way Vacation | (719)520-6897 |
| Floodplain Administrator | (719)327-2898 |
| EPC Public Health | (719)578-3199 |
| CDPHE Air Quality Division | (303)692-3100 |
| CDPHE Water Quality Division | (303)692-3500 |
| Cherry Creek Basin Water Quality Authority | (303)239-5400 |

Traffic Information

| Traffic Impact Study (ECM Appendix B)* |  |
| --- | --- |
| Full TIS | ADT > 1,000 or Pk Hr > 100 |
| Intermediate TIS | ADT < 1,000 or Pk Hr < 100 |
| Traffic Memo | ADT ≤ 500 or Pk Hr ≤ 50 |
| No TIS | ADT < 100 or Pk Hr < 10 |

*An approved TIS that has been prepared in the last three years may be revised or updated where a proposed access is changed or a change in the proposed action may result in a new trip generation that exceeds the original trip generation estimates. An amendment letter is required.

**If the original TIS is older than three years, an entirely new TIS shall be prepared.

***Any submitted TIS is subject to third party review based on size and complexity, in which additional fees will be assessed to the applicant.

The applicant is responsible for obtaining any necessary approvals for impacts within other jurisdictions. It is recommended that the applicant approach the jurisdiction early in the development process for any additional requirements. If the proposal is impacted by an El Paso County Department of Public Works project, coordination with DPW is required.

Pavement Condition Index

The Pavement Condition Index (PCI) is a numerical rating system that measures the condition of a road's pavement on a scale of 0 to 100. A score of 100 indicates a newly paved road, while 0 is the worst possible condition. The PCI is calculated based on the type and severity of distresses observed on the pavement surface, such as cracking, rutting, scaling, and spalling. Other factors that can affect the PCI score include Pavement age, Climate and precipitation, Traffic loads, and maintenance.

Drainage Report/Plan Information

Drainage Basin Planning Study (DBPS) – May be required with a very large development

Master Development Drainage Plan (MDDP) – Required with phased development greater than 10 acres (may be required with Sketch Plan) (DCM Section 4.2)

Drainage Letter Report – With a Re-plat, Minor Sub or Plot Plan (DCM Section 4.5) where a final drainage report has previously been approved.

Preliminary Drainage Report (PDR) – Required with a Preliminary Plan (DCM Section 4.3)

Final Drainage Report (FDR) – Required with a Final Plat (DCM Section 4.4)

Drainage Master Plan/Floodplain Information

If it is undetermined at this time if an adopted Drainage Basin Planning Study (DBPS) or Master Plan exists in this area, the applicant is responsible for researching the appropriate information.

If the subdivision application impacts the FEMA floodplain, a FEMA Letter of Map Revision may be required. If the site is in or near a floodplain, contact the Regional Floodplain Administrator for allowed floodplain uses and procedural requirements. It is the Applicant’s responsibility to research the effects and implications of developing in or near a floodplain, including limited uses, floodplain development permits, geotechnical, wetland and wildlife studies, structural requirements, flood insurance and potential future floodplain mapping updates. FEMA's FIRM maps may be out of date or at a low level of accuracy.

If the site lies within the Cherry Creek Basin Water Quality Authority area and is not rural density, the applicant should discuss any concerns related to the development with the CCBWQA’s consultant.

If the site lies within the Fountain Creek watershed, the District has authority over the floodplain impacts and is advisory to the County on uses outside of the floodplain. The applicant should discuss any concerns related to the development with District staff.

Detention/Water Quality PCM & Downstream Conveyance Information

Per ECM Chapter 3.2.8.B: “The proposed project or developed land use shall not change historical runoff values, cause downstream damage, or adversely impact adjacent properties.”

The project must provide an acceptable method of storm drainage conveyance and may be required to construct (or contribute an equitable share to the construction of) a storm conveyance or collector system.

If the site conveys storm drainage flows through or across an adjacent private property, then the applicant is responsible for obtaining off-site drainage easements in accordance with the Engineering Criteria Manual Section 3.3.3.K. Any offsite easements necessary for the development shall be recorded prior to County Plan approvals.

If on-site facilities are required, the applicant will be required to provide access and drainage easements in accordance with Section 11.2.2 of the Drainage Criteria Manual.

If a water quality and/or detention facility is applicable, some or all of the following documents may be required: Operation and Maintenance (O&M) Manual, Private Detention Basin / Stormwater Quality BMP (ie: PCM) Maintenance Agreement, and Stormwater Detention and Infiltration (SDI) Form.

For post-construction water quality PCM requirements, see Appendix I of the Engineering Criteria Manual. If/when the site disturbs >1ac of non-excluded areas, a stormwater quality treatment facility (ie: PCM) will be needed for the runoff from the non-excluded areas. Exclusions can be found in our PCM Applicability Form and ECM Appendix I.7.1.

State Engineer’s requirements regarding dams and water rights apply. Jurisdictional/Non-Jurisdictional dam construction forms are typically required for detention facilities. If applicable, provide proof of submittal of the State’s “Notice of Intent to Construct a Non-Jurisdictional Water Impoundment Structure” form.

Grading, Erosion, and Sediment Control Information

The Grading and Erosion Control (GEC) Plan must be prepared in accordance with Drainage Criteria Manual Vol. 2, Chapter 3 and the County’s GEC Checklist.

An Erosion and Stormwater Quality Control Permit (ESQCP) and Permanent Control Measure are required for the following conditions per ECM Appendix I.4:

- Construction activities that result in land disturbance of greater than or equal (≥) to one acre, or

- Construction activities that result in less than (<) one acre if the activity is part of a larger “Common Plan of Development or Sale” (ECM Chapter 1.22) that would disturb one acre or more of ground surface

An ESQCP may be required as determined by the ECM Administrator for the following conditions per ECM Appendix I.4:

- Construction activities that are located in or adjacent to a Sensitive Area, or

- Cut and/or fill greater than (>) 500 cubic yards of soil

See the three ESQCP exclusions listed in ECM Chapter 5.6.3.

An application for an ESQCP shall be accompanied by the following:

| - Drainage Report - ESQCP Form - Financial Surety - GEC Plan and GEC Checklist | - PCM Applicability Form - Stormwater Management Plan (SWMP) and SWMP Checklist - O&M Manual (for sites requiring a PCM) - PCM Maintenance Agreement (for sites requiring a PCM |
| --- | --- |

A Builder’s Erosion and Stormwater Quality Control Permit (BESQCP) is required for residential building lots with < 1 acre of disturbed areas and does not meet any of the criteria for an ESQCP listed above.

Construction activities that disturb >1ac of soil are required by the Environmental Protection Agency (EPA) to obtain a Construction Stormwater Permit through the Colorado Department of Public Health & Environment (CDPHE) Water Quality Control Divisions. For projects with soil disturbances >1ac but <25ac, a County Construction Activity Permit is required, contact El Paso County’s Public Health Department for further information regarding the permit. All land development activities greater than twenty-five (25) acres or with construction duration longer than six (6) months must obtain an Air Pollution Emission Notice and Emissions Permit through CDPHE’s Air Quality Control Division.

Deviations

All engineering designs and studies shall be performed in conformance with adopted codes, standards and criteria. Any deviations are to be formally identified and requested in writing, with justification provided per ECM Section 5.8. All deviation requests must be approved by the ECM Administrator prior to submitting the application for review, or delays in the review and additional fees may result. The Applicant must submit adequate justification for consideration of the request(s).

Public Improvements Information

If public improvements are required, the applicant will be required to enter into a Subdivision Improvement Agreement (SIA) or a development agreement (if the project is not a subdivision) with the County. Refer to the El Paso County Land Development Code for information on the SIA and the ECM for the required format of the associated Financial Assurance Estimate. All forms are available online.

Construction drawings for the required public improvements must be reviewed and approved by PCD and the County Engineer. All Construction drawing submittals shall adhere to the criteria set forth in the ECM. A construction plan review fee will be assessed when the plans are submitted for review.

Geotechnical reports for earthwork and pavement designs must be reviewed and approved.  All submittals must adhere to the criteria set forth in the ECM.

Any work within the Right of Way will require a permit from El Paso County Department of Public Works.

Any Fire cistern(s) and mailbox kiosks need to be shown on the construction drawings.

Metro District Fees

| Woodmen Road Metro District | Central Marksheffel Metro District | Lorson Ranch Metro District | Constitution Heights Metro District |
| --- | --- | --- | --- |
| - Property in district pays no fee - After December 31, 2020 no property in the Amended Service Area shall be permitted to join the WRMD, and shall participate fully in the Fee Program. - As of January 1, 2021 and until either bonds retire or December 31, 2027, whichever occurs sooner, El Paso County must pay a portion of the Road Impact Fee it collects from property in the Amended Service Area that receives final plat approval to WRMD. | - Property in district pays no fee - District granted $2.9 million in Fee Program Credit - Fee Program will pay annual credit reimbursements to District, or can sell credits | - Property in district pays full fee - District granted $4.1 million in credit - Credits used toward future development, can apply for reimbursement, can sell credits | - Property in district pays full fee - District granted $3.1 million in credit - Used toward future development, can sell credits, NO reimbursement - County will collect building permit fees for reimbursement of PPRTA improvements to Marksheffel Road from property within District and some property outside District – Credit given toward Impact Fee for amount of building permit fee paid. |

*Properties within the Meridian Crossing subdivision are not subject to the county wide road impact fee.