LEGAL DESCRIPTION

Those portions of Atwood Drive and Vollmer Road as platted in Southwood Subdivision, recorded in Plat Book J-2 at Page 15 of the records of El Paso County, being a portion of the East 1/2 of Section 21 and the Southwest 1/4 of Section 22, Township 11 South, Range 65 West of the 6th P.M., County of El Paso, State of Colorado, more particularly described as follows:

BASIS OF BEARINGS: Bearings are based upon the North line of the Southeast Quarter of said Section 21, monumented on the West end with a #6 rebar and 2 1/2" aluminum cap stamped "PLS 16154", and monumented on the East end with a 1" pipe, and is assumed to bear N 89%%D20'59" E, a measured distance of 2633.31 feet.

BEGINNING at the East Quarter Corner of said Section 21, said point also being the Southwest corner of Lot 3, Rampart View, as recorded under Plat Book P-3, Page 28;

thence N 89°54'52" E, along the South line of said Lot 3, a distance of 30.00 feet to a point on the East right-of-way line of said Vollmer Road;

thence S 00°00'43" E along said East right-of-way line, a distance of 382.58;

thence S 70°16'24" W, a distance of 47.32 feet;

thence S 42°17'00" W, a distance of 22.97 feet to a point on the West right-of-way line of said Vollmer Road;

thence N 00°00'43" W along said West right-of-way line, a distance of 385.15 feet to a point on the South right-of-way line of Atwood Drive;

thence S 89°20'59" W along said South right-of-way line, a distance of 1278.13 feet;

thence N 49°10'40" E, a distance of 93.01 feet to a point on the North right-of-way line of said Atwood Drive;

thence N 89°20'59" E along said North right-of-way line, a distance of 1237.74 feet to a point on the West line of said Lot 3, Rampart View;

thence S 00°00'21" W, along said West line a distance of 30.00 feet to the POINT OF BEGINNING.

Containing a calculated area of 99,158 square feet (2.276 acres) of land, more or less.