Parcel 2, Traits at Timberline East being a portion of the Southwest Quarter of Section 22, Township 12 South, Range 65 east of the 6TH P.M. as described in Warranty Deed, filed for record on November 15, 2016 under Reception No. 216132384 in the Office of Clerk and Recorder, County of El Poso, State of Colorado, being more particularly described as follows:

Commencing at the southeast corner of the Southwest Quarter (SWI/4) of said Section 22; thence South 88 degrees 38 minutes 37 sec	West along the south line of the Southeast Quarter of the Southwest Quarter (SEI/4, SWI/4), a distance of 30.00 feet;

thence North 0 degrees 18 minutes 17 seconds West, a distance of 852.29 feet to the Point of Beginning of the parcel of land herein described; thence South 89 degrees 41 minutes 56 seconds West, a distance of 1283.66 feet;

thence South 38 degrees 17 minutes 19 seconds West, a distance of 182.67 feet; thence South 56 degrees 32 minutes 31 seconds West, a distance of 489.24 feet; thence South 36 degrees 59 minutes 01 seconds West, a distance of 517.38 feet;

thence South 47 degrees 35 minutes 42 seconds West, a distance of 105.23 feet to a point on the south line of the Southwest Quarter of the Southwest

Quarter (SWI/4, SWI/4) of said Section 22;

thence South 88 degrees 38 minutes 56 seconds West along said south line. a distance of 149.37 feet; thence North 27 degrees 23 minutes 34 seconds West, a distance of 611.28 feet;

thence North 57 degrees 24 minutes 01 seconds East, a distance of 235.36 feet; thence North 35 degrees 00 minutes 25 seconds East, a distance of 167.38 feet; thence North 9 degrees 56 minutes 46 seconds East, a distance of 270.03 feet; thence North 40 degrees 09 minutes 47 seconds East, a distance of 310.61 feet; thence North 89 degrees 39 minutes 18 seconds East, a distance of 356.07 feet; thence North 79 degrees 15 minutes 47 seconds East, a distance of 276.17 feet; thence North 61 degrees 52 minutes 38 seconds East, a distance of 227.40 feet; thence North 30 degrees 23 minutes 46 seconds West, a distance of 264.58 feet; thence North 69 degrees 36 minutes 18 seconds East, a distance of 141.59 feet; thence North 1 degrees 18 minutes 16 seconds East, a distance of 150.40 feet; thence North 89 degrees 41 minutes 56 seconds East, a distance of 1247.78 feet;

thence South 0 degrees 18 minutes 04 seconds East, a distance of 997.65 feet to the Point of Beginning.

Together with that track of as described:

A portion of the Parcel 1 South described in General Warranty Deed filed for record October 1, 2025 under Reception No. 225085554 located in a portion of the Southwest Quarter of Section 22, Township 12 South, Range 65 West of the 6TH P.M., County of El Paso, State of Colorado, being more particularly described as follows:

Commencing at the Southwest corner of Section 22; thence North 0 degrees 37 minutes 10 seconds West along the west line of said Section 22 (the Basis of Bearings for this legal description is the west line of said Parcel 1 South, monumented at the south end by a found 1 1/2” aluminum cap, PLS 34977 and at the north end by a found No. 5 rebar & orange cap, PLS 32820, assumed to bear North 3 degrees 17 minutes 05 seconds East), 40.00 feet to a point on the south line of Stimple Subdivision No. 1, filed for record May 14, 2025 under Reception No. 225715515 also being the north right—of—way line of Arroya Lane; thence North 88 degrees 42 minutes 35 seconds East along the south line of said Stimple Subdivision and the north Right—of—Way line of Arroya Lane, 158.11 feet to the southeast corner of said Stimple Subdivision; thence continuing along the north Right—of—Way line of Arroya Lane North 88 degrees 34 minutes 05 seconds East, 104.80 feet; thence deporting the north Right—of—Way line of Arroya Lane and running along the east line of said Parcel 1 South the following five (5)  courses:

thence North 27 degrees 23 minutes 34 seconds West, 566.68 feet;

thence North 57 degrees 24 minutes 10 seconds East, 235.42 feet;

thence North 35 degrees 00 minutes 33 seconds East, 167.35 feet;

thence North 9 degrees 56 minutes 59 seconds East, 270.05 feet;

thence North 40 degrees 09 minutes 31 seconds East, 310.59 feet to the Point of Beginning of the parcel herein described;

thence North 40 degrees 09 minutes 31 seconds East, 188.04 feet;

thence North 69 degrees 35 minutes 06 seconds East, 426.85 feet;

thence South 44 degrees 13 m1nutes 21 seconds East, 271.53 feet;

thence South 61 degrees 51 m1nutes 35 seconds West, 94.39 feet;

thence South 79 degrees 16 m1nutes 58 seconds West, 276.19 feet;

thence South 89 degrees 39 m1nutes 20 seconds West, 356.09 feet to the Point of Beginning. Containing a calculated area of 118,737 square feet or 2.7258 acres, more or less.