FALCON FIRE PROTECTION DISTRICT

Fire Prevention Division

7030 Old Meridian Road

Falcon, Colorado 80831

Business Number: 719-495-4050     Business Fax: 719-495-3112

Lieutenant/Fire Inspector II           Curtis L. Kauffman

Email: ckauffman@falconfirepd.org                     Cell #: 719-641-2139

Date: May 15, 2025

To: EDARP

C/O: Lacey Dean

Planning & Community Development

2882 International Circle, Suite 110

Colorado Springs, CO 80910

Phone Number: 719-520-7943 x7943

Email: LaceyDean2@elpasoco.com

Project Name: 7735 Maverick Road-ALQ Special Use

File Prefix: AL-Special Use

Parcel/s: 5304003006

File Number: AL2511

7735 Maverick Road, Colorado Springs CO 80908

This letter serves as special use comments. You are responsible for complying with all section’s, codes and references with the fire code based on the 2021 International Fire Code (IFC).

PLEASE READ THIS ENTIRE DOCUMENT, YOU ARE RESPONSIBLE FOR EVERYTHING CONTAINED HERIN!

The following are ACTION ITEMS that need to be completed prior to the fire final.

- Address’s shall be visible from the road/driveway and by the front door of both house doors.

Section 505.1. Address Identification. New and existing buildings shall have approved address numbers, building numbers or approved building identification placed in a position that is plainly legible and visible from the street or road fronting the property. These numbers shall contrast with their background. Address numbers shall be Arabic numbers or alphabetical letters. Numbers shall be a minimum of five (5) inches (127 mm) high with a minimum stroke width of one half (0.5) inch (12.7 mm). Where access is by means of a private road and the building cannot be viewed from the public way, a monument, pole or other means of signage shall be used to identify the structure.”

- The driveway shall be passable for fire apparatus. It shall be able to withstand 75,000 load pounds. Minimum width is 12-feet and minimum height shall be 13-feet & six-inches.

The following are ATTENTION ITEMS that apply to this review.

- Follow the Colorado State Forest Service for all mitigation guidelines.

Attention Contractor/Applicant:

YOU ARE RESPONSIBLE TO COMPLETE THE FOLLOWING ITEMS.

Please complete the following items:

- Review the comment letter from Falcon Fire Protection District. You are responsible for complying with the entire fire code as amended and any applicable NFPA’s referenced.

- Ensure all Action Items have been addressed.

- Permits – Insure all applicable permits are obtained from the Pikes Peak Regional Building Department (PPRBD).

This project is approved by the Falcon Fire Protection District as long as all action and attention items are followed.

Please let me know if there are any questions and/or concerns. You may contact me via the email at the top of the letter.

Sincerely,

Curtis L. Kauffman

Curtis L. Kauffman

Fire Prevention Division

Lieutenant/Fire Inspector II