WILDFIRE PROTECTION PLAN

November 24, 2025

CRYSTAL PARK SITE # 179

250 Summit Rd.

Manitou Springs, CO.

Developed for Owners: Terri and David Rauh

Prepared by: David Rauh and Greg Wickham (Builder)

SUBMITTED TO  :

El Paso Land Use Office

Crystal Park Fire Department

Crystal Park Architectural Control Committee

WILDFIRE PROTECTION PLAN

This Wildfire Protection Plan is submitted to the Land Use Office of El Paso County, Colorado, County Zoning Department, the Crystal Park Fire Department and the Crystal Park Architectural Control Committee to satisfy their requirements in planning for wildfire protection and mitigation in connection with a new home on Site # 179 in Crystal Park Sub 2.

The Property is Subject to this Plan

This plan is presented in connection with construction of a new single family home on the subject site. The site plan, including contour lines with the 5’ and 30’ setback lines from the proposed home,  is attached.

The application for a building permit is in process.

This plan determines the required defensible space around the proposed home site and it identifies the vegetation and forest characteristics in the surrounding area.

Colorado State Forest Service has presented “The Home Ignition Zone :  a guide to preparing your home for wildfire and creating a defensible space”. An inspection of the site and the surrounding area has been conducted and the attached site plan has been developed in accordance with this guide and it’s fact sheets as they relate to the property’s Three Defensible Zones.

The Homeowners and Builder all recognize that it is the responsibility of the Owners to protect their property and persons from Wildfire.

Structural Ignitability

The use of fire-resistant materials is highly encouraged in Crystal Park, and they are being used in the construction of the subject home. The roof will be constructed using Class “A” material. The front and rear patios will be constructed of concrete and stone.

The exterior walls are to be constructed of stucco and stone. The roof eaves and fascia are to be constructed of cement board (Hardi-Board). The windows shall be aluminum frame.

The Owners and Builder have consulted the CFSF Fire Wise Construction, Site Design and Building Materials at www.csfs.colostate.edu, and the Home Ignition Guide Defensible Space.

Defensible Space is the area around a home that has been modified to reduce fire hazard. It will give your home a good chance against a wildfire. This involves developing three zones around the structure on the property. Within these Defensible Zones, the following will be accomplished in accordance with the guidelines of “The Home Ignition Zone” by Colorado State Forest Service.

Defensible Space Management Zone 1 : 0 – 5 ft.  This zone is designated to prevent flames from coming in direct contact with the structure. Use nonflammable, hard surface materials in this zone, such as rock, gravel, cement, bare earth or stone/concrete pavers.

CRITICAL STEPS

- Remove all flammable vegetation, including shrubs, slash, mulch and other woody debris

- Do not store firewood or other combustible materials inside this zone.

- Prune tree branches hanging over the roof and remove all fuels within 10’ of the chimney

- Regularly remove all pine needles and other debris from the roof, deck and gutters

- Rake and dispose of pine needles, dead leaves, mulch, and other organic debris within 5’ of all decks and structures. Farther than 5’ from structures, raking material will not significantly reduce the likelihood of ignition and can negatively affect other trees.

- Do not use space under decks for storage.

Defensible Space Management Zone 2 : 5’ – 30 ft.  This zone is designed to give an approaching fire less fuel, which will reduce it’s intensity as it gets closer to any structures.

CRITICAL STEPS

- Mow grasses to 4 inches tall or less

- Avoid large accumulations of surface fuels such as logs, branches, slash and mulch.

- Remove enough trees to create at least 10 feet of space between crowns. Measure from the outermost branch on one tree to the nearest branch on the next tree.

- Small groups of two or three trees may be left in some areas on zone 2. Spacing of 30 feet should be maintained between remaining tree groups to ensure fire does not jump from one group to another.

- Remove ladder fuels under remaining trees. This is any vegetation that can bring fire from the ground up to taller fuels.

- Prune tree branches to a height of 6-10 feet from the ground or a third of the total height of the tree, whichever is less.

- Remove stressed, diseased dead or dying trees and shrubs. This reduces the amount of vegetation available to burn and improves forest health.

- Common ground junipers should be removes whenever possible because they are highly flammable and tend to hold a layer of flammable material beneath them.

- You can keep isolated shrubs in zone 2, as long as they are not growing under trees. Keep shrubs at least 10 feet away from tree branches.

- Periodically prune and maintain shrubs to prevent excessive growth. Remove dead stems often.

- Spacing between clumps of shrubs should be at least 2 ½ times their mature height. Each clump should have a diameter no more than twice the mature height of the vegetation. Example : for shrubs that grow 6 feet tall, space clumps 15 feet apart or more (measure from the crowns of the vegetation clumps). Each clump of these shrubs should not exceed 12 feet in diameter.

Horizontal spacing recommendations are minimums and can be increased to reduce potential fire behavior, especially on slopes. Consult a forestry, fire, or resource professional for guidance with spacing on slopes.

Defensible Space Management Zone 3 : 30 – 100 feet  This zone focuses on mitigation that keeps fire on the ground, but it’s also a space to make choices that can improve the health of the forest. Healthy forests include trees of multiple ages, sizes and species, where adequate growing room is maintained over time.

If the distance of 100 feet stretches beyond your property lines, it’s encouraged to work with adjoining property owners to complete an appropriate defensible space.

If your house is on steep slopes or has certain topographical considerations, this zone may be larger.

CRITICAL STEPS

- Mowing grasses is not necessary in zone 3.

- Watch for hazards associated with ladder fuels. The chance of a surface fire climbing into trees is reduced in a forest where surface fuels are widely separated, and low tree branches are removed.

- Tree crown spacing of 6-10 feet is suggested. Consider creating openings or meadows between small clumps of trees so fire has to transition to the ground to keep moving.

- Any approved method of slash treatment is acceptable in this zone. This includes removing, piling and burning, lop and scatter, or mulching. Lop-and-scatter treatments should be minimized in favor of treatments that reduce the amount of woody material in the zone. The further this material is from the home, the better.

Other Recommendations

Water Supply

Crystal Park has several cisterns installed around the Park for the express purpose of firefighting. The Owner’s will verify the cistern locations.

Recommendations for Specific Forest Types

The above recommendations refer primarily to Ponderosa Pine, Douglas Fir and mixed conifer ecosystems.

Implementation

This plan is to be used and in place by the time construction of the home is completed, and implementation will begin during construction.  This does not include the Defensible Space Management of adjacent sites owned by others, nor common ground owned by Crystal Park. The Owners will work with other property owners to assist in their management of defensible space when necessary. Any nearby areas of defensible space that is Crystal Park Common Area will be addressed by the Owners to the Crystal Park Forestry Committee by contacting the Crystal Park Office at 719-685-9729.

The Owners Implementation Plan shall be inspected by the Regional State Forester of the Colorado State Forest Service upon completion of the house construction. The State Forester must approve the Fire Mitigation work and adherence to this mitigation plan submitted before the Owners occupy the house.