LOT 2, BLOCK 1, PINE HILLS EXCEPTION PARCEL

That portion of Lot 2, Block 1, Pine Hills, recorded October 11, 1956 in Plat Book Y at Page 44 in the records of El Paso County, said parcel being in the Northeast quarter of Section 22, Township 11 South, Range 67 West of the 6th P.M., County of El Paso, State of Colorado, described as follows:

Bearings are based upon the common line between Lots 2 and 3, Block 3, Pine Hills, recorded October 11, 1956 in Plat Book Y at Page 44 in the records of El Paso County, monumented on the East end with a 1 1/4" steel pipe, 0.1' below grade and monumented on the West end with a 1" steel pipe, 1.0' below grade and bears N 89°35'00" W, a field measured distance of 1092.00 feet.

COMMENCING at the Southwest corner of said Lot 2;

thence S 89°35'00" E along the South line of said Lot 2, a distance of 278.66 feet to the POINT OF BEGINNING;

thence leaving said South line N 00°17'33" E, a distance of 38.00 feet;

thence S 89°35'00" E and parallel with said South line, a distance of 573.98 feet;

thence S 00°17'33" W, a distance of 25.21 feet;

thence S 89°35'00" E and parallel with said South line, a distance of 125.72 feet;

thence S 00°17'33" W, a distance of 12.79 feet to a point on said South line;

thence N 89°35'00" W along said South line, a distance of 699.70 feet to the POINT OF BEGINNING.

Containing a calculated area of 23,419 square feet, (0.538 acres) of land, more or less.