FLYING HORSE NORTH PUD 2025

LEGAL DESCRIPTION: WEST PARCEL

A PARCEL OF LAND BEING A PORTION OF SECTIONS 34 AND 35, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE EAST LINE OF SECTION 35, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, BEING MONUMENTED AT THE NORTH END BY A 2 1/2" ALUMINUM CAP STAMPED “22564” AND THE SOUTH END BY A 2 1/2" ALUMINUM CAP STAMPED “9132”, IS ASSUMED TO BEAR S00°14'34”E, A DISTANCE OF 5269.38 FEET.

COMMENCING AT THE NORTHEAST CORNER OF SECTION 35, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, SAID POINT BEING THE POINT OF BEGINNING

THENCE S00°14’34"E, ON THE EAST LINE OF SAID SECTION 35, A DISTANCE OF 523.85 FEET TO A POINT ON CURVE;

THENCE ON THE ARC OF A CURVE TO THE LEFT WHOSE CENTER BEARS S33°01'51”W, HAVING A DELTA OF 38°24’48”, A RADIUS OF 535.00 FEET AND A DISTANCE OF 358.69 FEET TO A POINT OF TANGENT;

THENCE S84°37'03W, A DISTANCE OF 175.44 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 11°13’59", A RADIUS OF 615.00 FEET AND A DISTANCE OF 120.57 FEET TO A POINT OF TANGENT;

THENCE N84°08’58'W, A DISTANCE OF 684.98 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 25°13‘51”, A RADIUS OF 615.00 FEET AND A DISTANCE OF 270.82 FEET TO A POINT OF TANGENT;

THENCE N58°55’07W, A DISTANCE OF 166.51 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE LEFT HAVING A DELTA OF 31°18'40”, A RADIUS OF 535.00 FEET AND A DISTANCE OF 292.37 FEET TO A POINT OF TANGENT;

THENCE S89°46'13”W, A DISTANCE OF 1674.58 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE LEFT HAVING A DELTA OF 24°52’43", A RADIUS OF 1960.00 FEET AND A DISTANCE OF 851.06 FEET TO A POINT OF TANGENT;

THENCE S64°53’30”W, A DISTANCE OF 459.47 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 21°22'27", A RADIUS OF 1040.00 FEET AND A DISTANCE OF 387.97 FEET TO A POINT OF TANGENT;

THENCE S86°15’57”W, A DISTANCE OF 692.41 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE LEFT HAVING A DELTA OF 51°05'38”, A RADIUS OF 535.00 FEET AND A DISTANCE OF 477.09 FEET TO A POINT OF TANGENT;

THENCE S35°10’18W, A DISTANCE OF 291.93 FEET TO A POINT OF CURVE;

THENCE ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 53°07'49", A RADIUS OF 615.00 FEET AND A DISTANCE OF 570.29 FEET TO A POINT OF TANGENT;

THENCE S88°18’07”W, A DISTANCE OF 160.75 FEET TO A POINT ON THE EASTERLY RIGHT OF WAY LINE OF STATE HIGHWAY 83;

THENCE N01°41'53”W, ON SAID EASTERLY RIGHT OF WAY LINE A DISTANCE OF 90.00 FEET TO THE SOUTHWESTERLY CORNER OF LOT 1 AS PLATTED IN WESCOTT FIRE STATION NO. 3, RECORDED UNDER RECEPTION NO. 212T13192 RECORDS OF EL PASO COUNTY, COLORADO;

THENCE ON THE SOUTHERLY, EASTERLY AND NORTHERLY BOUNDARY OF SAID LOT 1 THE FOLLOWING (5) FIVE COURSES;

- N88°18'07”E, A DISTANCE OF 165.75 FEET TO A POINT OF CURVE;

- ON THE ARC OF A CURVE TO THE LEFT HAVING A DELTA OF 54°10'43", A RADIUS OF 460.00 FEET AND A DISTANCE OF 434.97 FEET TO A POINT OF REVERSE CURVE;

- ON THE ARC OF A CURVE TO THE RIGHT HAVING A DELTA OF 15°19'05", A RADIUS OF 560.00 FEET AND A DISTANCE OF 149.72 FEET TO A POINT ON CURVE;

- N38°00'00”W, A DISTANCE OF 141.67 FEET;

- S88°20'00'W, A DISTANCE OF 587.56 FEET TO A POINT ON THE EASTERLY RIGHT OF WAY OF SAID STATE HIGHWAY 83;

THENCE ON SAID EASTERLY RIGHT OF WAY THE FOLLOWING (3) THREE COURSES;

- N01°41’53’W, A DISTANCE OF 446.49 FEET;

- N00°02’53”W, A DISTANCE OF 245.49 FEET TO A POINT ON CURVE;

- ON THE ARC OF A CURVE TO THE RIGHT WHOSE CENTER BEARS S87°06’46”E, HAVING A DELTA OF 07°31'38”, A RADIUS OF 1380.65 FEET AND A DISTANCE OF 181.38 FEET TO A POINT ON CURVE, SAID POINT BEING THE SOUTHWESTERLY CORNER OF HIGH FOREST RANCH FILING NO. 1, RECORDED UNDER RECEPTION NO. 201036672, SAID POINT ALSO BEING ON THE NORTH LINE OF THE NORTHWEST QUARTER OF THE NORTHEAST QUARTER OF SECTION 34, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO;

THENCE N89°54’54"E, ON THE SOUTHERLY BOUNDARY OF SAID HIGH FOREST RANCH FILING NO. 1, AND SAID NORTH LINE OF THE NORTHWEST QUARTER OF THE NORTHEAST QUARTER OF SECTION 34, A DISTANCE OF 584.61 FEET TO THE EAST SIXTEENTH CORNER OF SAID SECTION 34; THENCE S89°57'36”E, ON THE NORTH LINE OF THE NORTHEAST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 34 AND CONTINUING ON SAID SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 1, A DISTANCE OF 1319.30 FEET TO THE NORTHEAST CORNER OF SAID SECTION 34; THENCE N89°46’13”E, CONTINUING ON SAID SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 1 AND ON THE NORTH LINE OF THE NORTHWEST QUARTER OF SECTION 35, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, A DISTANCE OF 2660.56 FEET TO THE NORTH QUARTER CORNER OF SAID SECTION 35;

THENCE N89°45’50"E, CONTINUING ON SAID SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 1, ON THE NORTH LINE OF THE NORTHEAST QUARTER OF SAID SECTION 35 AND THE SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 2, RECORDED UNDER RECEPTION NO. 202134767, A DISTANCE OF 2048.33 FEET;  THENCE ON SAID SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 2, THE FOLLOWING (5) FIVE COURSES:

- N44°21'15”E, A DISTANCE OF 120.12 FEET;

- N27°42'44"E, A DISTANCE OF 30.37 FEET;

- N83°51'56"E, A DISTANCE OF 62.76 FEET;

- S79°32’21"E, A DISTANCE OF 69.45 FEET;

- S46°40’23”E, A DISTANCE OF 153.82 FEET TO A POINT ON SAID NORTH LINE OF THE NORTHEAST QUARTER OF SAID SECTION 35;

THENCE N89°48'10”E, ON SAID SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 2 AND SAID NORTH LINE OF THE NORTHEAST QUARTER OF SECTION 35, A DISTANCE OF 270.47 FEET TO THE POINT OF BEGINNING;

CONTAINING A CALCULATED AREA OF 70.926 ACRES.

LEGAL DESCRIPTION: EAST PARCEL

A PARCEL OF LAND BEING ALL OF SECTION 36 TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, AND A PORTION OF SECTIONS 30 AND 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE SIXTH PRINCIPAL MERIDIAN, ALL IN EL PASO COUNTY, COLORADO, BEING MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS: THE SOUTH LINE OF THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 3, TOWNSHIP 12 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, BEING MONUMENTED AT THE WEST END BY A 2” ALUMINUM CAP STAMPED ”24964" AND THE EAST END BY A 2 1/2" ALUMINUM CAP STAMPED “CCES LLC PLS 30118”, IS ASSUMED TO BEAR S89°51’39"E, A DISTANCE OF 1316.82 FEET.

COMMENCING AT THE NORTHWEST CORNER OF SECTION 36, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, SAID POINT BEING THE POINT OF BEGINNING;

THENCE N89°06’04”E, ON  THE  SOUTHERLY BOUNDARY OF HIGH FOREST RANCH FILING NO. 2, RECORDED UNDER RECEPTION NO. 202134767, RECORDS OF EL PASO COUNTY, COLORADO AND THE  NORTH LINE OF NORTHWEST  QUARTER  OF  THE NORTHWEST QUARTER OF SECTION 36, TOWNSHIP 11 SOUTH, RANGE 66 WEST  OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO,  A DISTANCE OF 1332.12 FEET TO THE SOUTHEASTERLY  CORNER  OF  SAID  HIGH  FOREST  RANCH FILING NO. 2, SAID POINT BEING THE WEST  SIXTEENTH CORNER  OF SAID SECTION 36; THENCE N89°07’00”E,  ON  THE  NORTH  LINE OF  THE NORTHEAST  QUARTER  OF THE NORTHWEST QUARTER OF SAID SECTION 36, A DISTANCE OF 1331.92 FEET TO THE NORTH QUARTER CORNER OF SAID SECTION 36; THENCE N89°01'18"E, ON THE NORTH LINE OF THE NORTHWEST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 36, A DISTANCE OF 1331.92 FEET TO THE EAST SIXTEENTH CORNER OF SAID SECTION 36; THENCE N89°03'58”E, ON THE NORTH LINE OF THE NORTHEAST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 36, A DISTANCE OF 1332.09 FEET TO THE NORTHEAST CORNER OF SAID SECTION 36; THENCE N89°06'20”E, ON THE NORTH LINE OF THE NORTHWEST QUARTER OF THE NORTHWEST QUARTER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO, A DISTANCE OF 1474.13 FEET TO THE WEST SIXTEENTH CORNER OF SAID SECTION 31; THENCE N00°08’36”E, ON THE WEST LINE OF THE SOUTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 30, TOWNSHIP 11 SOUTH, RANGE 65 WEST  OF THE SIXTH PRINCIPAL MERIDIAN, EL  PASO COUNTY, COLORADO,  A DISTANCE OF 1325.48 FEET TO THE SOUTHWEST SIXTEENTH CORNER OF SAID SECTION 30; THENCE N89°03’20”E, ON THE NORTH LINE OF SAID SOUTHEAST QUARTER OF THE SOUTHWEST QUARTER  OF  SECTION  30,  A  DISTANCE  OF  920.27  FEET  TO THE SOUTHWEST CORNER OF THE EASTERLY TWELVE (12) ACRES OF THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SAID SECTION 30; THENCE N00°08'15”E, ON THE WEST LINE OF SAID EASTERLY (12) TWELVE ACRES OF THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 30, A DISTANCE OF 1326.26 FEET TO THE NORTHWESTERLY CORNER OF SAID EAST (12) TWELVE ACRES OF THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 30, SAID POINT BEING ON THE NORTH LINE OF SAID NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 30 (HELD MONUMENTS DEPICTED ON LAND SURVEY PLAT DEPOSITED UNDER RECEPTION NO.  91000488 BY BERGE- BREWER & ASSOCIATES, INC ON JULY 30, 1991); THENCE N89°01'31“E, ON THE NORTH LINE OF SAID NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 30, A DISTANCE OF 399.42 FEET TO THE CENTER QUARTER OF SAID SECTION 30; THENCE N00°08’48”E, ON THE WEST LINE OF THE NORTHEAST QUARTER OF SAID SECTION 30, A DISTANCE OF 2604.74 FEET TO A POINT ON THE SOUTHERLY BOUNDARY OF A PARCEL OF LAND DESCRIBED IN A DOCUMENT RECORDED UNDER RECEPTION NO. 210081316: THENCE ON SAID BOUNDARY, THE FOLLOWING (3) COURSE:

- N88°58’45”E, A DISTANCE OF 2270.00 FEET;

- S71°21’27"E, A DISTANCE OF 29.72 FEET;

- N88°58’45”E, A DISTANCE OF 299.96 FEET TO A POINT ON A LINE 30.00 FEET WEST OF AND PARALLEL TO THE EAST LINE OF SAID NORTHEAST QUARTER OF SECTION 30;

THENCE S00°00'48”W, ON SAID PARALLEL LINE, A DISTANCE OF 2595.64 FEET TO A POINT ON A LINE 30.00 FEET WEST OF AND PARALLEL TO THE EAST LINE OF THE SOUTHEAST QUARTER SAID SECTION 30; THENCE S00°00'53"W, ON SAID PARALLEL LINE, A DISTANCE OF 2656.67 FEET TO A POINT ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF THE SOUTHEAST QUARTER OF SAID SECTION 30; THENCE S89°04’37"W, ON SAID SOUTH LINE, A DISTANCE OF 1290.01 FEET TO THE EAST SIXTEENTH CORNER OF SAID SECTION 31;

THENCE S00°00’11”E, ON THE EAST LINE OF THE NORTHWEST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 31, A DISTANCE OF 1326.67 FEET TO THE NORTHEAST SIXTEENTH CORNER OF SAID SECTION 31; THENCE N89°08'21”E, ON THE NORTH LINE OF THE SOUTHEAST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 31, A DISTANCE OF 1289.57 FEET TO  A POINT ON A LINE 30.00 FEET WEST OF AND PARALLEL TO THE EAST LINE OF SAID SOUTHEAST QUARTER OF THE NORTHEAST QUARTER OF SECTION 31; THENCE S00°00’54"W, ON SAID PARALLEL LINE, A DISTANCE OF 1328.09 FEET TO A POINT ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF THE NORTHEAST QUARTER OF SAID SECTION 31, SAID POINT BEING ON THE NORTHERLY BOUNDARY OF COUNTRY VIEW ESTATES, RECORDED UNDER RECEPTION NO. 99011204; THENCE S89°11'15”W, ON SAID SOUTH LINE AND THE NORTHERLY BOUNDARY OF SAID COUNTRY VIEW ESTATES AND ITS WESTERLY EXTENSION, A DISTANCE OF 2608.28 FEET TO THE CENTER QUARTER CORNER OF SAID SECTION 31; THENCE S89°11'00”W, ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF THE NORTHWEST QUARTER OF SAID SECTION 31, A DISTANCE OF 1320.84 FEET TO THE CENTER-WEST SIXTEENTH CORNER OF SAID SECTION 31; THENCE S00°00’34”W, ON THE EAST LINE OF THE NORTHWEST QUARTER OF THE SOUTHWEST QUARTER OF SAID SECTION 31, A DISTANCE OF 1329.16 FEET TO THE SOUTHWEST SIXTEENTH CORNER OF SAID SECTION 31, SAID POINT BEING ON THE NORTHERLY BOUNDARY OF PALMER DIVIDE, RECORDED UNDER RECEPTION NO. 205084216; THENCE S89°24’17"W, ON THE SOUTH LINE OF SAID NORTHWEST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 31 AND SAID NORTHERLY BOUNDARY OF PALMER DIVIDE AND ITS WESTERLY EXTENSION, A DISTANCE OF 1440.81 FEET TO THE SOUTH SIXTEENTH CORNER OF SAID SECTION 31; THENCE S00°28’30“E, ON THE EAST LINE OF THE SOUTHEAST QUARTER OF THE SOUTHEAST QUARTER OF SAID SECTION 36, A DISTANCE OF 1323.57 FEET TO THE SOUTHEAST CORNER OF SAID SECTION 36, SAID POINT ALSO BEING THE NORTHEASTERLY CORNER OF EDMONDS SUBDIVISION, RECORDED IN PLAT BOOK H- 3 AT PAGE 60; THENCE S89°20'59”W, ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF SAID SECTION 36, THE NORTHERLY BOUNDARY OF SAID EDMONDS SUBDIVISION AND THE NORTHERLY BOUNDARY OF CATHEDRAL PINES SUBDIVISION FILING NO. 2, RECORDED UNDER RECEPTION NO. 205164426, A DISTANCE OF 2674.51 FEET TO THE SOUTH QUARTER CORNER OF SAID SECTION 36; THENCE S89°20'35”W, ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 30, CONTINUING ON SAID NORTHERLY BOUNDARY OF CATHEDRAL PINES SUBDIVISION FILING NO. 2 AND ON THE NORTHERLY BOUNDARY OF CATHEDRAL PINES SUBDIVISION FILING NO. 3, RECORDED UNDER RECEPTION NO. 206712390, A DISTANCE OF 2674.51 FEET TO THE SOUTHWEST CORNER OF SAID SECTION  36; THENCE N00°14’34”W, ON THE WEST LINE OF SAID SECTION 36, A DISTANCE OF 5269.38 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 1346.825 ACRES.

INCLUSION PARCELS:

LEGAL DESCRIPTION: (A PORTION OF 5100000291) ASSESSORS NUMBER

THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 31 IN TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE 6TH P.M., COUNTY OF EL PASO, STATE OF COLORADO, MORE PARTICULARLY DESCRIBED AS FOLLOWS:

BASIS OF BEARINGS THE SOUTH ONE OF THE SOUTHEAST QUARTER OF THE NORTHWEST QUARTER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE 6TH P.M. COUNTY OF EL PASO STATE OF COLORADO, BEING MONUMENTED ON THE WESTERLY END BY A 2” ALUMINUM CAP STAMPED PLS 18465” AT GROUND LEVEL AND AT THE EASTERLY END BY A 2” ALUMINUM CAP STAMPED “PLS 18465” AT GROUND LEVEL, BEING ASSUMED TO BEAR N89°11’00”W ADDO 1320.84 FEET.

COMMENCING AT THE CENTER-WEST ONE SIXTEENTH CORNER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE 6TH P.M. COUNTY OF EL PASO STATE OF COLORADO SAID POINT BEING THE POINT OF BEGINNING; THENCE N89°11’00”E ON THE SOUTH LINE OF THE SOUTHEAST QUARTER OF SAID SECTION 31, A DISTANCE OF 1320.84 FEET TO THE CENTER QUARTER CORNER OF SAID SECTION 31; THENCE S00°01’43”E ON THE EAST OF THE NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SAID SECTION 31 A DISTANCE OF 1,334.25 FEET TO A POINT ON THE SOUTH LINE OF SAID NORTHEAST QUARTER OF THE SOUTHWEST QUARTER SECTION 31; THENCE S89°24’17”W ON SAID SOUTH LINE A DISTANCE OF 1321.66 FEET TO THE WEST LINE OF SAID NORTHEAST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 31; THENCE N00°00’34”E ON SAID WEST LINE A DISTANCE OF 1,329.16 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 1,759,384 SQUARE FEET OR 40.390 ACRES.

LEGAL DESCRIPTION: (5100000437) ASSESSORS NUMBER

THE SOUTHWEST QUARTER OF THE SOUTHWEST QUARTER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE 6TH P.M., COUNTY OF EL PASO STATE OF COLORADO, EXCEPT THAT PORTION AS DISCLOSED IN THE BOUNDARY LINE AGREEMENT RECORDED NOVEMBER 15, :2004 AT RECEPTION NO. 204188565, MORE PARTICULARLY DESCRIBED AS FOLLOWS;

COMMENCING AT THE NORTHWEST CORNER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE SIXTH PRINCIPAL MERIDIAN AND THE NORTHEAST CORNER OF SECTION 36, TOWNSHIP 11 SOUTH, RANGE 66 WEST OF THE SIXTH PRINCIPAL MERIDIAN, EL PASO COUNTY, COLORADO; THENCE S00°28'17" E, ON THE EAST LINE OF SAID SECTION 36, A DISTANCE OF 3970.82 FEET TO THE SOUTH SIXTEENTH CORNER OF SECTION 31, TOWNSHIP 11 SOUTH, RANGE 65 WEST OF THE SIXTH PRINCIPAL MERIDIAN, SAID POINT BEING THE POINT OF BEGINNING; THENCE N89°24'17"E, ON THE SOUTH LINE OF THE NORTHWEST QUARTER OF THE SOUTHWEST QUARTER OF SAID SECTION 31, A DISTANCE OF 1403.03 FEET TO A POINT ON A LINE DESCRIBED IN A BOUNDARY LINE AGREEMENT RECORDED UNDER RECEPTION NO. 204188565; THENCE S02°42'03"W, ON SAID LINE DESCRIBED IN A BOUNDARY LINE AGREEMENT RECORDED UNDER RECEPTION NO. 204188565 A DISTANCE OF 1330.04 FEET TO A POINT ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 31; THENCE S89°35'20"W, ON THE SOUTH LINE OF THE SOUTHWEST QUARTER OF SAID SECTION 31, A DISTANCE OF 1329.35 FEET TO THE SOUTHWEST CORNER OF SAID SECTION 31 SAID POINT ALSO BEING THE SOUTHEAST CORNER OF SAID SECTION 36; THENCE N00°28'30"W, ON THE EAST LINE OF THE SOUTHEAST QUARTER OF THE SOUTHEAST QUARTER OF SAID SECTION 36, A DISTANCE OF 1323.57 FEET TO THE POINT OF BEGINNING.

CONTAINING A CALCULATED AREA OF 41.580 ACRES..