PUDSP252 WATER REVIEW COMMENTS

SP17012, the original Flying Horse North PUD/Preliminary Plan, received a recommendation for a conditional finding of water supply sufficiency from this office dated July 15, 2018. That review contemplated the following uses and annual water demands:

283 residential lots		170.2 AF (including indoor use, irrigation, stock watering)

Golf course			180 AF

Open space			27.8 AF

Total				378 AF

Thus far, 6 plats have been recorded, comprising 203 lots. As described in the letter of intent, this application seeks to increase the total number of lots originally reviewed in Flying Horse North from 283 to 299. By my calculations, however, the total number of lots is proposed to increase to 301, resulting in an increase of 18 lots.

FHN1:		81		FHN6:		  1

FHN2:		  1		FHN7:		50

FHN3:		51		FHN8:		47

FHN4:		48		Total		98

FHN5:		21

Lot 35 replat:	  1

Total		203

In Table 3 of the Water Resources Report, the applicant estimates the total annual water demand of Flying Horse North to be 438.31 AF, an increase of 60.31 AF (16%) from the original water demand reviewed.

Per LDC § 8.4.7 (E)(2), the PCD Director is to make an administrative determination as to whether a proposed change in water supply for a subdivision is substantial. It is my recommendation, based on the percentage increase in estimated water demand, the proposed change is substantial.

Whether determined to be substantial or not, I recommend that the following documentation be provided for my office to make a recommendation regarding approval of the change in water supply:

- A favorable recommendation from the State Engineer’s Office, which will at a minimum entail providing them with the information requested in their letter dated May 28, 2025.

- Please add a discussion to the Water Resources Report that explains the difference between the estimated demand for the various categories of water use in the 2017 application and that provided with this application.