Judge Orr Rd RV Park Stormwater Action Items

- Over-excavate at outlet pipes to act as sediment trap; install rock check dam or other control measure for additional protection

- Get survey/elevation shots of culverts and existing ditches along Judge Orr Rd

- Print SWMP, site map, control measure details and place on site for review. Refer to CDPHE COR400000 permit and Aug 22 email from El Paso County. Ensure self-inspections are being conducted per state and county requirements.

- Swale E-E not completed per plan; install ECB per plans and details or have engineer resubmit plans for approval to riprap swale instead of ECB

- Final grade slopes around outlet pipes and implement stabilization measures on slopes; install perimeter control measures at top of slope to prevent erosion

- Complete concrete trickle channel in eastern pond

- Complete riprap rundowns at N forebay and S forebay in eastern pond

- Implement stabilization measures per plan in eastern pond

- Install restrictor plate, orifice plate, and gaskets at outlet structure in eastern pond

- Install  riprap per plan at emergency spillway K-K, spillway swale L-L, and outlet swale J-J

- Grade per plan and install ECB in swales B-B and D-D

- Install ECB and erosion bales at swale A-A per plans (west ditch/pond)

- Install riprap rundown in western ditch/pond per plan

- Outlet pipe in Swale E-E missing flared end section